# ECM AP 10 Template — Section-by-Section Comparison (CHE vs USA)

CHE file: ECM AP 10 KERAMIK MIT WAB VIEW E.docx USA file: ECM AP 10 Ceramic MIT WAB VIEW_US_FCA Muttenz_11 2025.docx

Content extracted programmatically from the Word documents. Each subsection is shown in a two-column table (CHE left, USA right) with a difference assessment below.

## Section 1 — Principal Characteristics

CHE: 21 rows (single-column table). USA: 22 rows (single-column table).

### 1.1 Working Principle

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| High-Efficiency Agitator Bead Mill using the “Patented” DYNO-ACCELERATOR grinding principle | High-Efficiency Agitator Bead Mill using the “Patented” DYNO-ACCELERATOR grinding principle |
| The DYNO-ACCELERATOR ensure powerful hydraulic movement of the grinding beads and guarantee high & uniform energy input | The DYNO-ACCELERATOR ensures powerful hydraulic movement of the grinding beads and guarantees high & uniform energy input |
| DYNO-ACCELERATOR are designed to achieve the narrowest possible Particle Size Distribution | DYNO-ACCELERATOR is designed to achieve the narrowest possible Particle Size Distribution |
| Grinding media separation by means of the unique combination of the DYNO-ACCELERATOR and the new WAB separation system ADVANCED which permits the use of grinding beads from 0.1 mm - 1.25 mm | Grinding media separation by means of the unique combination of the DYNO-ACCELERATOR and the new WAB separation system ADVANCED which permits the use of grinding beads from 0.1 mm - 1.25 mm |
| Most efficient heat transfer allows for superior cooling, permitting the processing of heat-sensitive products | Efficient heat transfer allows for superior cooling, permitting the processing of heat sensitive products |
| Lowest power consumption due to highly efficient energy input (low carbon footprint) | Low power consumption due to highly efficient energy input (low carbon footprint) |

Difference: Modest — Grammar corrections (plural→singular verb agreement: “ensure”→“ensures”, “guarantee”→“guarantees”, “are designed”→“is designed”). Superlatives toned down (“Most efficient”→“Efficient”, “Lowest”→“Low”). Hyphen removed from “heat-sensitive”→“heat sensitive”.

### 1.2 Unique Process Flexibility

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| Suitable for dispersion and wet grinding of low to high viscosity products | Suitable for dispersion and wet grinding of low to high viscosity products |
| Wide range of energy input allows dispersing/grinding of a broad range of products at its optimum parameters | Wide range of energy input allows dispersing/grinding of a broad range of products at optimum parameters |
| Suitable for both, passage- and circulation processes | Suitable for both, passage and recirculation processes |
| Unique batch size flexibility | Unique batch size flexibility |

Difference: Minor — Same changes as UBM: “at its optimum”→“at optimum”; “passage- and circulation”→“passage and recirculation”.

### 1.3 Design

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| Designed to ensure optimal ergonomics & user-friendly handling | Designed to ensure optimal ergonomics & user-friendly handling |
| Reduced noise emission by mill motor placed inside the machine housing | Reduced noise emission by mill motor placed inside the machine housing |
| Easy operation & adjustment of operation parameters | Easy operation & adjustment of operating parameters |
| Built-in safety features | Built-in safety features |
| (not present) | Equipment is designed in accordance with applicable UL safety requirements but is not UL certified and will not bare the UL mark |

Difference: Modest — Same changes as UBM: “operation”→“operating”; UL safety disclaimer row added.

### 1.4 Cleaning & Maintenance

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| Self Cleaning effect of grinding chamber | Self-cleaning effect of grinding chamber |
| Short Cleaning Times using the smallest quantities of cleaning fluids, resulting in — faster product turnarounds — reduced costs for cleaning fluids & recycling — less exposure to solvents by operator and environment | Short cleaning times using the smallest quantities of cleaning fluids, resulting in — faster product turnaround — reduced costs for cleaning fluids & recycling — less exposure to solvents by operator and environment |
| Fast exchange of the screen (without the need to open the grinding container) | Fast exchange of the screen (without the need to open the grinding container) |
| Easy accessibility to all parts reduces handling time to a minimum | Easy accessibility to all parts reduces handling time |
| Double mechanical seal from WAB for long life and low maintenance costs | Double mechanical seal from WAB for long life and low maintenance costs |

Difference: Minor — Capitalisation normalised (“Self Cleaning”→“Self-cleaning”, “Cleaning Times”→“cleaning times”). “turnarounds”→“turnaround”. “to a minimum” removed from accessibility line.

## Section 2 — General Technical Data

### 2.1 Conformity — Left Side (Labels)

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| EC Declaration of Conformity: | Conformity/ Protection: |
| Explosion protection of the machine: | (merged into single row) |
| Protection: | (merged into single row) |

### 2.1 Conformity — Right Side (Values)

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| covered by the Directive 2006/42/EC on machinery, Annex II A, and the ATEX directive 2014/34/EU, Annex VIII | General purpose/not rated — Rated area/Class I/Div I |
| II 2 G Ex h IIB T3 Gb | (see above — single combined row) |
| IP 55 (acc. IEC 60529) | (see above — single combined row) |

Difference: Major — Identical pattern to UBM: CHE has three rows with EU/ATEX declarations. USA collapses to a single row with US classification options. Unlike UBM (which had a question mark), the ECM USA version lists both options without a question mark (“General purpose/not rated” and “Rated area/Class I/Div I”) suggesting two configuration choices.

### 2.2 Electrical Specifications — Left Side (Labels)

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| Operating voltage: | Operating voltage: |
| (not present) | Installed power: |
| Control voltage: | Control voltage: |

### 2.2 Electrical Specifications — Right Side (Values)

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| 400 V, 50 Hz (3 phases) | 460 V, 60 Hz (3 phases) |
| (not present) | 50 HP (37 kW) |
| 24 VDC | 24 V DC |

Difference: Major — Same pattern as UBM: voltage/frequency changes (400V/50Hz→460V/60Hz), “Installed power” row added, “VDC”→“V DC”.

### 2.3 Grinding Unit — Left Side (Labels)

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| Grinding cylinder volume: | Grinding cylinder volume: |
| Max. working pressure: | Max. working pressure: |
| Working temperature: | Working temperature: |
| Product inlet: | Product inlet/outlet: |
| Product outlet: | (merged with above) |

### 2.3 Grinding Unit — Right Side (Values)

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| 9.0 liter | 9.0 liter |
| 4.0 bar | 4.0 bar |
| 5 °C - 85 °C | 5 °C to 85 °C |
| Tri-Clamp connection Ø 50.5 mm acc. DIN 32676 for tube acc. DIN 11866 line B DN/OD Ø 33.7 mm with hose connection Ø 31.8 mm and with PTFE seal | Tri-Clamp connection Ø 2” with Ø 1.25” hose connection and with PTFE seal |
| (same as inlet) | (merged with above) |

Difference: Modest — Same pattern as UBM: dash→“to” in temperature; DIN metric specs replaced with imperial equivalents; inlet/outlet merged into one row. Note: ECM CHE has no “Active grinding cylinder volume” row (unlike UBM CHE which had it).

### 2.4 Cooling Water Circulation System

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| Central cooling water connection for the cooling of the grinding unit and the rinsing liquid device: With external thread connection G ½” | Central cooling water connection for the cooling of the grinding unit and the rinsing liquid device: With external thread connection G ½” |
| Cooling water information | Cooling water information |
| Flow rate: 1500 – 2000 l/h | Flow rate: 1500 – 2000 l/h |
| Pressure drop (with free outlet): 2.5 – 4.0 bar | Pressure drop (with free outlet): 2.5 – 4.0 bar |
| Max. cooling water pressure: 6.0 bar | Max. cooling water pressure: 6.0 bar |
| Cooling water temperature range: -15 °C – +50 °C | Cooling water temperature range: -15 °C to +50 °C |
| Recommended cooling water temperature: Below 10 °C | Recommended cooling water temperature: Below 10 °C |

Difference: Minor — Only “–”→“to” in temperature range. Note: ECM cooling system is simpler than UBM (single central connection vs separate grinding container + buffer pressure unit subsections).

### 2.5 Machine Housing

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| Paint: 2-Component paint of high quality, chemical resistant (in the colours grey, blue) | Paint: 2-Component paint of high quality, chemical resistant (in the colours grey, blue) |
| Dimensions L x W x H (mm): Approx. 1520 x 1000 x 1920 | Dimensions L x W x H: Appr. 60 x 40 x76 in (1520 x 1000 x 1920 mm) |
| Weight: Approx. 1200 - 1300 kg | Weight: Appr. 2645 - 2866 lb (1200 - 1300 kg) |

Difference: Modest — Same pattern as UBM: imperial primary with metric in parentheses. “Approx.”→“Appr.” Note a formatting issue in USA: “x76” has no space.

## Section 3 — Machine Execution

### 3.1 Grinding Unit

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| 1× Cooling cylinder in stainless steel including a removing device for fast & simple access to the milling chamber / DYNO-ACCELERATOR due to the ergonomic design | 1× Stainless steel cooling cylinder with a removing device for fast & simple access to the milling chamber / DYNO-ACCELERATOR |
| 1× Grinding cylinder made of wear resistant silicon carbide | 1× Grinding cylinder made of wear resistant silicon carbide |
| 1× Front cover of stainless steel with product outlet connection and drain plug | 1× Stainless steel front cover with product outlet connection and drain plug |
| 1× Set of DYNO-ACCELERATOR made of zirconium oxide / silicon carbide | 1× Set of DYNO-ACCELERATORs made of zirconium oxide / silicon carbide |
| 1× WAB Separation system ADVANCED with central deaeration and screen disc of zirconium oxide | 1× WAB Separation system ADVANCED with central deaeration and screen disc of zirconium oxide |
| 1× Cylinder Screen 0.25 mm, pure nickel, hard chromium plated | 1× Cylinder Screen 0.25 mm, pure nickel, hard chromium plated |
| 1× Set of spacers of zirconium oxide / silicon carbide including electrically conductive-/FDA compliant taper seals | 1× Set of spacers of zirconium oxide / silicon carbide including electrically conductive-/FDA compliant taper seals |
| (empty row) | (empty row) |
| Product contacted O-rings of perfluorelastomer/FEP-O-Seal | Product contacted O-rings of Perfluor/FEP-O-Seal |
| Remaining parts in contact with the grinding beads in the wear area of zirconium oxide / silicon carbide | Remaining parts in contact with the grinding beads in the wear area of zirconium oxide / silicon carbide |

Difference: Minor — Word-order rephrasing (“Cooling cylinder in stainless steel”→“Stainless steel cooling cylinder”; “Front cover of stainless steel”→“Stainless steel front cover”). “due to the ergonomic design” removed from first row. Plural “DYNO-ACCELERATORs” in USA. “perfluorelastomer”→“Perfluor” (same as UBM).

### 3.2 Shaft Sealing

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| 1× Double mechanical seal from WAB — Product side and atmosphere side sliding rings made of wear resistant silicon carbide with wear disc of zirconium oxide; fully sealed with perfluorelastomer O-rings | 1× Double mechanical seal from WAB — Product side and atmosphere side sliding rings made of wear resistant silicon carbide with wear disc of zirconium oxide; fully sealed with Perfluor O-rings |
| 1× Rinsing and pressurising of the shaft seal by means of separately driven, stainless steel gear pump type ZR 4030 — Motor 0.25 kW, 3 x 400 V, 50 Hz, 1000 min⁻¹ — II 2 G Ex eb IIC T3 Gb, IP 55 — Operating pressure adjustable with stainless steel control valve — Rinsing liquid reservoir with cooling coil of stainless steel | 1× Rinsing and pressurizing of the shaft seal by means of separately driven, stainless steel gear pump type ZR 4030 — Motor .33 HP (0.25 kW), 3 x 460 V, 60 Hz, 1000 rpm — Stainless steel control valve to adjust operating pressure — Rinsing liquid reservoir with cooling coil of stainless steel |
| 1× Separate leakage container for the rinsing liquid | 1× Separate leakage container for the rinsing liquid |

Difference: Major — Motor specs changed: voltage (400V→460V), frequency (50Hz→60Hz), speed units (min⁻¹→rpm), power adds HP (“.33 HP”). Full ATEX classification (Ex eb IIC T3 Gb, IP 55) removed. “pressurising”→“pressurizing” (UK→US). “perfluorelastomer”→“Perfluor”. Control valve sentence rephrased. Note: ECM USA uses “.33 HP” (missing leading zero) vs UBM’s “0.33 HP”.

### 3.3 Drive Unit

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| 1× Mill motor 37 kW, 3 x 400 V, 50 Hz, 1500 min⁻¹ — II 2 G Ex db eb IIB T4 Gb, IP 55, with 3 PTC probes | 1× Mill motor 50 HP (37 kW), 3 x 460 V, 60 Hz, 1500 |
| 1× Pulley and antistatic V-belt for DYNO-ACCELERATOR tip speed of 4.0 – 16.0 m/s | 1× Pulley and antistatic V-belt for DYNO-ACCELERATOR tip speed of 4.0 – 16.0 m/s |

Difference: Major — Same pattern as UBM: HP added as primary, voltage 400V→460V, frequency 50Hz→60Hz. ATEX classification and PTC probes removed. Note: USA text appears truncated — “1500” with no unit (missing “rpm”). Unlike UBM (1500→1750 rpm), ECM retains 1500 speed value.

### 3.4 Sensors and Control Unit

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| 1× Explosion protection of sensors according Directive 2014/34/EU concerning equipment and protective systems intended for use in potentially explosive atmospheres — II 2 G Ex ia/ib IIC T4-T6 Gb | (entire row removed) |
| Safety and control instruments consisting of: Two inductive proximity switches to prevent start of the mill when grinding container or WAB separation system is removed; Tube pressure transmitter in the mill inlet, with output signal 4-20 mA and tube shaped Tri-Clamp connection; Temperature transmitter (PT 100) in the mill outlet, with output signal 4-20 mA and pen sensor mounted in a robust protective tube; Two capacitive proximity switches for level control of the rinsing liquid; Tube pressure transmitter in the seal rinsing liquid circuit, with output signal 4-20 mA; Pressure gauge 0 - 6 bar with to control the pressure in the seal rinsing circuit, mounted in the machine housing; Instruments connected in a junction box for connection to electric circuit II 2 G Ex ia/ib IIC T4-T6 Gb, without relays | 1× Safety and control instruments consisting of: Two inductive proximity switches to prevent start of the mill when grinding container or WAB separation system is removed; Contact pressure gauge (MIN/MAX) 0 - 6 bar at the product inlet with inductive contacts and Tri-Clamp connection; Bimetal thermometer (MAX) 0 - 100 °C with pen sensor mounted in a robust protective tube; Two capacitive proximity switches for level control of the rinsing liquid; Contact pressure gauge (MIN/MAX) 0 - 6 bar with inductive contacts to control the pressure in the seal rinsing circuit; Instruments connected in a junction box for connection to electric circuit, without relays |
| 1× Mill Control Unit WAB VIEW Pro, mounted in the machine housing (inside wired on terminals) for control voltage 24 VDC — Touchdisplay (12” TFT) — Explosion protection ATEX Zone 1 and Zone 21, IP 66 — Processor Intel ATOM E3940, RAM 8 GB, Hard disc 128 GB — Software: Windows 10 IoT Enterprise 2021 LTSC, WinCC Runtime Advanced V17 — USB interface for transmission of process parameters — EMERGENCY STOP button — Explosion protection II 2 G Ex db eb IIC T6 Gb, IP 65 | 1× Mill Control Unit WAB VIEW mounted in the machine housing (inside wired on terminals) for control voltage 24 V DC — Touchdisplay (7” TFT) — USB interface for transmission of process parameters on USB-Stick — EMERGENCY STOP button mounted separately in the machine housing |

Difference: Major — Three significant changes: 1. ATEX sensor row entirely removed (same as UBM). 2. Instruments row substantially different: CHE uses transmitters with 4-20 mA output signals (tube pressure transmitter, PT 100 temperature transmitter); USA uses simpler instruments (contact pressure gauges, bimetal thermometer). ATEX junction-box classification removed. The CHE instruments are more sophisticated (transmitter-based with electronic output) while USA uses mechanical gauge equivalents. 3. Control unit downgraded: WAB VIEW Pro→WAB VIEW; 12” TFT→7” TFT; all hardware specs removed (processor, RAM, hard disc); all software specs removed (Windows, WinCC); all ATEX classifications removed (two separate declarations). The USA version is dramatically simplified.

### 3.5 Machine Housing & Support Material

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| 1× Machine housing in painted steel | 1× Painted steel machine housing |
| 1× Stainless steel collecting dish for the grinding beads | 1× Stainless steel collecting dish for the grinding beads |
| 1× Toolbox for dismantling of the mill parts | 1× Toolbox for dismantling of the mill parts |
| 1× Set of torque wrenches | 1× Set of torque wrenches |
| 1× Funnel made of stainless steel for filling of grinding beads | 1× Stainless steel funnel for filling of grinding beads |
| 1× Operational Manual in English | 1× Operational Manual in English |

Difference: Minor — Word-order rephrasing only (same patterns as UBM: “Machine housing in painted steel”→“Painted steel machine housing”; “Funnel made of stainless steel”→“Stainless steel funnel”).

### 3.6 Price — Machine Execution

ECM has two price lines (zirconium oxide and silicon carbide variants):

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| Price Machine Execution zirconium oxide: CHF 242’730.00 | Price Machine Execution zirconium oxide: USD 404,324.00 |
| Price Machine Execution silicon carbide: CHF 209’030.00 | Price Machine Execution silicon carbide: USD 347,236.00 |

Difference: Major — Currency CHF→USD. Thousand separator apostrophe→comma. Factors: 404,324 / 242,730 ≈ 1.666× (zirconium oxide); 347,236 / 209,030 ≈ 1.661× (silicon carbide). These are close but not identical to each other or to the UBM factor (~1.694×).

## Section 4 — Product Pump

CHE title: “Product Pump DM 2500”. USA title: “Product Pump” (model designation removed).

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| 1× Pneumatic driven membrane pump | 1× Pneumatic driven membrane pump |
| Explosion protection of the product pump: Directive 2014/34/EU concerning equipment and protective systems intended for use in potentially explosive atmospheres — II 2 GDc IIB T80°C | (ATEX declaration entirely removed) |
| Pneumatic driven membrane pump to be installed beside the DYNO-MILL ECM-AP 10. | Pneumatic driven membrane pump to be installed beside the DYNO-MILL ECM-AP 10. |
| Pump specification | Pump specification |
| Range of capacity 100 – 2500 l/h | Range of capacity 100 – 2500 l/h |
| Stainless steel housing | Stainless steel housing |
| Membranes of PTFE | Membranes of PTFE |
| Seals of PTFE | Seals of PTFE |
| Needle valve for pressure regulation G ¼” | Needle valve for pressure regulation G ¼” |
| Pulsation damper, stainless steel housing, membrane of PTFE | Pulsation damper, stainless steel housing, membrane of PTFE |
| Solenoid valve and connecting box to be fitted on site | Solenoid valve and connecting box to be fitted on site |
| (not present) | Tubing between feed tank, pump, mill, and discharge is not included |

Difference: Modest — ATEX declaration (II 2 GDc IIB T80°C) removed. Tubing-not-included disclaimer added. All pump specifications are otherwise identical (this is a pneumatic pump, so no electrical motor specs to change). Model designation “DM 2500” removed from section title.

### 4.1 Price — Product Pump

| CHE (ECM AP 10 KERAMIK) | USA (ECM AP 10 Ceramic) |
| --- | --- |
| Price Product Pump: CHF 11’066.00 | Price Product Pump: USD 18,746.00 |

Difference: Major — Currency CHF→USD. Factor: 18,746 / 11,066 ≈ 1.694× (matches UBM factor exactly).

## Summary of Differences

| Subsection | Assessment | Key Changes |
| --- | --- | --- |
| 1.1 Working Principle | Modest | Verb agreement fixes, superlatives toned down |
| 1.2 Unique Process Flexibility | Minor | “circulation”→“recirculation” (same as UBM) |
| 1.3 Design | Modest | UL safety disclaimer added (same as UBM) |
| 1.4 Cleaning & Maintenance | Minor | Capitalisation normalised, “to a minimum” removed |
| 2.1 Conformity | Major | EU/ATEX declarations→US classification options |
| 2.2 Electrical Specifications | Major | 400V/50Hz→460V/60Hz, installed power added |
| 2.3 Grinding Unit | Modest | DIN metric→imperial, inlet/outlet merged |
| 2.4 Cooling Water Circulation | Minor | Dash→“to” in temperature range only |
| 2.5 Machine Housing | Modest | Imperial primary units with metric in parens |
| 3.1 Grinding Unit | Minor | Word-order rephrasing, “perfluorelastomer”→“Perfluor” |
| 3.2 Shaft Sealing | Major | Motor specs changed, ATEX removed |
| 3.3 Drive Unit | Major | Motor specs changed, ATEX removed, possible truncation |
| 3.4 Sensors and Control Unit | Major | ATEX removed, transmitters→gauges, WAB VIEW Pro→VIEW, 12”→7” TFT, all HW/SW specs removed |
| 3.5 Machine Housing & Support | Minor | Word-order rephrasing only |
| 3.6 Price — Machine Execution | Major | CHF→USD, two price lines, factors ~1.661–1.666× |
| 4 Product Pump (full) | Modest | ATEX removed, tubing disclaimer added, model name removed; pump specs otherwise identical (pneumatic) |
| 4.1 Price — Product Pump | Major | CHF→USD, factor ~1.694× |

## Cross-Product Comparison: ECM vs UBM Patterns

| Pattern | UBM | ECM | Same? |
| --- | --- | --- | --- |
| Section 1 grammar fixes | yes | yes | yes |
| UL safety disclaimer added | yes | yes | yes |
| “passage- and circulation”→“passage and recirculation” | yes | yes | yes |
| Conformity: ATEX→US classification | yes (with ?) | yes (no ?) | ~yes |
| Electrical: 400V/50Hz→460V/60Hz | yes | yes | yes |
| Installed power row added | yes | yes | yes |
| Product I/O: DIN metric→imperial, merged | yes | yes | yes |
| Cooling water: nearly identical | yes | yes | yes |
| Machine housing: imperial primary | yes | yes | yes |
| Page size change | A4→Letter | A4→A4 | no |
| ATEX removal across sections 3-4 | yes | yes | yes |
| Motor specs: HP primary, TEFC, rpm | yes | yes (but possible truncation) | ~yes |
| “perfluorelastomer”→“Perfluor” | yes | yes | yes |
| Word-order: “X of stainless steel”→“Stainless steel X” | yes | yes | yes |
| 3.4 Control unit change | minor (WAB BASIC) | major (Pro→VIEW, 12”→7”, all specs removed) | no |
| Section 4 pump type | Gear pump (major changes) | Membrane pump (minor changes) | no |
| Tubing disclaimer added | yes | yes | yes |
| Price factor — section 3 | ~1.694× | ~1.661–1.666× | no |
| Price factor — section 4 | ~1.694× | ~1.694× | yes |

### Key Findings

- Most changes are generic across product lines — the same text edits, ATEX removal, motor-spec transformations, unit conversions, and UL disclaimer apply to both UBM and ECM.

- Product-line-specific differences exist in:

  - Page size: UBM changes A4→Letter; ECM stays A4.

  - Control unit: UBM has WAB BASIC (simple); ECM has WAB VIEW Pro→VIEW (significant downgrade with HW/SW spec removal).

  - Pump type: UBM has a gear pump (extensive spec changes); ECM has a pneumatic membrane pump (specs nearly identical, only ATEX removed).

  - Machine Execution price factor: UBM ~1.694×; ECM ~1.661–1.666× (suggesting multiplier selection may depend on product line or section content).

- Section 4 Product Pump price factor is ~1.694× for both product lines, while Section 3 Machine Execution factors diverge. This may indicate the price factor formula applies differently to higher-value sections or that manual adjustments were made.