# UBM 20 Template — Section-by-Section Comparison (CHE vs USA)

CHE file: UBM 20 FU_FU E.docx USA file: UBM 20 FCFC_US_FCA Muttenz_11 2025.docx

Content extracted programmatically from the Word documents. Each subsection is shown in a two-column table (CHE left, USA right) with a difference assessment below.

## Section 1 — Principal Characteristics

CHE: 25 rows (single-column table). USA: 27 rows (single-column table).

### 1.1 Working Principle

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| Agitator Bead Mill using the patented DYNO-DISC BC agitator discs | Agitator Bead Mill using the patented DYNO-DISC BC agitator discs |
| The DYNO-DISC BC accelerate the grinding media along a well-defined path to build effective grinding circuits | The DYNO-DISC BC accelerates the grinding media along a well-defined path to build effective grinding circuits |
| WAB grinding media separation by means of the combination of the DYNO-ROTOR BS (Bead Separation) and large WAB separation system UNIVERSAL. Permits the use of grinding beads from 0.2 mm – 2.5 mm | WAB grinding media separation by means of the combination of the DYNO-ROTOR BS (Bead Separation) and large WAB separation system UNIVERSAL. Permits the use of grinding beads from 0.2 mm – 2.5 mm |
| (not present) | Screens available: 0.07 mm for beads ≥ 0.2 mm, 0.13 mm for beads ≥ 0.4 mm, 0.25 mm for beads ≥ 0.8 mm, 0.40 mm for beads ≥ 1.2 mm, and 0.60 mm for beads ≥ 1.8 mm |
| Excellent cooling properties permit the processing of also heat-sensitive products | Excellent cooling properties permit the processing of heat-sensitive products |
| High product throughputs | High product throughput |
| Lower power consumption due to highly efficient energy input (low carbon footprint) | Lower power consumption due to highly efficient energy input (low carbon footprint) |

Difference: Modest — Grammar corrections (“accelerate” → “accelerates”), added screen-size options row (new content), removed “also” from heat-sensitive, singular “throughput”.

### 1.2 High Process Flexibility

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| Suitable for dispersion and wet grinding of low to high viscosity products | Suitable for dispersion and wet grinding of low to high viscosity products |
| Wide range of energy input allows dispersing/grinding of a broad range of products at its optimum parameters | Wide range of energy input allows dispersing/grinding of a broad range of products at optimum parameters |
| Quick and easy definition of stable processing parameters | Quick and easy definition of stable processing parameters |
| Suitable for both, passage- and circulation processes | Suitable for both, passage and recirculation processes |
| High batch size flexibility | High batch size flexibility |

Difference: Minor — “at its optimum” → “at optimum”; “passage- and circulation” → “passage and recirculation” (terminology change).

### 1.3 Design

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| Designed to ensure optimal ergonomics & user-friendly handling | Designed to ensure optimal ergonomics & user-friendly handling |
| Reduced noise emission by mill motor placed inside the machine housing | Reduced noise emission by mill motor placed inside the machine housing |
| Easy operation & adjustment of operation parameters | Easy operation & adjustment of operating parameters |
| Built-in safety features | Built-in safety features |
| (not present) | Equipment is designed in accordance with applicable UL safety requirements but is not UL certified and will not bare the UL mark |
| Robust beads collecting channel integrated in the machine housing | Robust bead collecting channel integrated into the machine housing |

Difference: Modest — Added UL safety disclaimer row (USA regulatory content), minor grammar edits (“operation” → “operating”, “beads” → “bead”, “in” → “into”).

### 1.4 Cleaning & Maintenance

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| Short cleaning times | Short cleaning times |
| Faster product turnarounds | Faster product turnaround |
| Fast exchange of the screen without the need to open the grinding chamber | Fast exchange of the screen without the need to open the grinding chamber |
| Wear-resistant design reducing service costs | Wear-resistant design reducing service costs |
| Easy accessibility to machine parts | Easy accessibility to machine parts |
| Double mechanical seal from WAB for long life | Double mechanical seal from WAB for long life |

Difference: Minor — “turnarounds” → “turnaround” (singular).

## Section 2 — General Technical Data

Each subsection is a labelled paragraph followed by a two-column table (label | value). The left column (labels) and right column (values) are compared separately where meaningful.

### 2.1 Conformity — Left Side (Labels)

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| EC Declaration of Conformity: | Conformity / Protection: |
| Explosion protection of the machine: | (merged into single row) |
| Protection: | (merged into single row) |

### 2.1 Conformity — Right Side (Values)

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| covered by the Directive 2006/42/EC on machinery, Annex II A, and the ATEX directive 2014/34/EU, Annex VIII | General purpose / not rated or Rated environment / Class I / Div 1? |
| II 2 G Ex h IIB T3 Gb | (see above — single combined row) |
| IP 55 (acc. IEC 60529) | (see above — single combined row) |

Difference: Major — CHE has three rows with EU/ATEX declarations (EC Directive 2006/42/EC, ATEX 2014/34/EU, explosion classification II 2 G Ex h IIB T3 Gb, IP 55). USA collapses to a single row with an unresolved question (“General purpose / not rated or Class I / Div 1?”). This section needs external input to finalize.

### 2.2 Electrical Specifications — Left Side (Labels)

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| Operating voltage: | Operating voltage: |
| (not present) | Installed power: |
| Control voltage: | Control voltage: |

### 2.2 Electrical Specifications — Right Side (Values)

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| 400 V, 50 Hz (3 phases) | 460 V AC, 60 Hz (3 phases) |
| (not present) | Approx. 50 HP (37 kW) |
| 24 VDC | 24 V DC |

Difference: Major — Voltage/frequency changes (400V/50Hz → 460V/60Hz), added “Installed power” row, minor formatting (“VDC” → “V DC”).

### 2.3 Grinding Unit — Left Side (Labels)

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| Grinding cylinder volume: | Grinding cylinder volume: |
| Active grinding cylinder volume: | Active grinding cylinder volume: |
| Max. working pressure: | Max. working pressure: |
| Working temperature: | Working temperature: |
| Product inlet: | Product inlet / outlet: |
| Product outlet: | (merged with above) |

### 2.3 Grinding Unit — Right Side (Values)

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| 19.6 litres | 19.6 litres |
| 17.4 litres | 17.4 litres |
| 4 bar | 4 bar |
| +5 °C – +85 °C | +5 °C to +85 °C |
| Tri-Clamp connection Ø 50.5 mm acc. DIN 32676 for tube acc. DIN 11866 line B DN/OD Ø 33.7 mm with hose connection Ø 31.8 mm and with PTFE seal | Tri-Clamp connection Ø 2”(or equivalent) with 1.25” hose barb fitting and PTFE gasket |
| Tri-Clamp connection Ø 50.5 mm acc. DIN 32676 for tube acc. DIN 11866 line B DN/OD Ø 33.7 mm with hose connection Ø 31.8 mm and with PTFE seal | (merged with above) |

Difference: Modest — Temperature range uses “to” instead of “–”. Product inlet/outlet merged into one row; DIN metric specs replaced with imperial equivalents (Ø 2”, 1.25” hose barb). “seal” → “gasket”.

### 2.4 Cooling Water Circulation Systems

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| Grinding container cooling | Grinding container cooling |
| Connection: Hose connection Ø 16 mm | Connection: Hose connection Ø 16 mm |
| Flow rate: 800 – 2000 l/h | Flow rate: 800 – 2000 l/h |
| Pressure drop (with free outlet): 0.3 – 1.6 bar | Pressure drop (with free outlet): 0.3 – 1.6 bar |
| Cooling cylinder temperature range: –15 °C – +50 °C | Cooling cylinder temperature range: –15 °C to +50 °C |
| Buffer pressure unit | Buffer pressure unit |
| Connection: With external thread connection G ½” | Connection: With external thread connection G ½” BSPP |
| Flow rate: 100 l/h | Flow rate: 100 l/h |
| Cooling water | Cooling water |
| Max. cooling water pressure: 6 bar | Max. cooling water pressure: 6 bar |
| Recommended cooling water temperature: Below 10 °C | Recommended cooling water temperature: Below 10 °C |

Difference: Minor — “–” → “to” in temperature range; added “BSPP” thread standard clarification.

### 2.5 Machine Housing

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| Paint: 2-Component paint of high quality, chemical resistant (in the colours grey, blue) | Paint: 2-Component paint of high quality, chemical resistant (in the colours grey, blue) |
| Dimensions L x W x H (mm): Approx. 1785 x 1025 x 1800 | Dimensions L x W x H: Appr. 70 x 40 x 71 in (1785 x 1025 x 1800 mm) |
| Weight: Approx. 1150 - 1250 kg | Weight: Appr. 2535 - 2755 lb (1150 - 1250 kg) |

Difference: Modest — Dimensions add inches as primary with mm in parentheses. Weight adds pounds as primary with kg in parentheses. “Approx.” → “Appr.” Page size is US Letter in USA vs A4 in CHE (handled at document level, not in this table).

## Section 3 — Machine Execution

### 3.1 Grinding Unit

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| 1× Cooling cylinder in stainless steel for fast access to the milling chamber due to the ergonomic design | 1× Cooling cylinder in stainless steel for fast access to the milling due to the ergonomic design |
| 1× Front cover, product inlet and product outlet of stainless steel | 1× Front cover, product inlet and product outlet of stainless steel |
| 1× Drain plug of hardened stainless steel mounted in the front cover | 1× Drain plug of hardened stainless steel mounted in the front cover |
| 1× Grinding cylinder made of hardened stainless steel | 1× Grinding cylinder made of hardened stainless steel |
| 1× Set of DYNO-DISC BC and DYNO-ROTOR BS made of hardened stainless steel | 1× Set of DYNO-DISC BC and DYNO-ROTOR BS made of hardened stainless steel |
| 1× Set of spacers of hardened stainless steel including special taper seals | 1× Set of spacers of hardened stainless steel including special taper seals |
| 1× WAB Separation system UNIVERSAL — Cylindrical slot screen made of stainless steel, PVD-coated, Gap width 0.25 mm, Screen cover made of stainless steel, hard chrome plated | 1× WAB Separation system UNIVERSAL — Cylindrical slot screen made of stainless steel, PVD-coated, Gap width 0.25 mm, Screen cover made of stainless steel, hard chrome plated |
| (empty row) | (empty row) |
| Remark to grinding unit: | (not present) |
| All remaining parts in contact with the grinding beads of hardened stainless steel | All remaining parts in contact with the grinding beads of hardened stainless steel |
| All product contacted O-rings of Viton/FEP-O-Seal | All product contacted O-rings of Viton/FEP-O-Seal |

Difference: Minor — “milling chamber” → “milling” (likely truncation/typo in USA). “Remark to grinding unit:” label row removed in USA. 11 rows CHE → 10 rows USA.

### 3.2 Shaft Sealing

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| 1× Double mechanical seal from WAB — Product side and atmosphere side sliding rings made of wear resistant Silicon Carbide; fully sealed with perfluorelastomer O-rings | 1× Double mechanical seal from WAB — Product side and atmosphere side sliding rings made of wear resistant Silicon Carbide; fully sealed with Perfluor O-rings |
| 1× Rinsing and pressurising of the shaft seal by means of separately driven, stainless steel gear pump type ZR 4030 — Motor 0.25 kW, 3 x 400 V, 50 Hz, 1000 min⁻¹ — II 2 G Ex eb IIC T3 Gb, IP 55 — Operating pressure adjustable with stainless steel control valve — Rinsing liquid reservoir with cooling coil of stainless steel | 1× Rinsing and pressurizing of the shaft seal by means of separately driven, stainless steel gear pump type ZR 4030 — Motor 0.33 HP (0.25 kW), 3 x 460 V, 60 Hz, 1200 rpm, TEFC — Operating pressure adjustable with stainless steel control valve — Rinsing liquid reservoir with stainless steel cooling coil |
| 1× Separate leakage container for the rinsing liquid | 1× Separate leakage container for the rinsing liquid |

Difference: Major — Motor specs fully changed: voltage (400V→460V), frequency (50Hz→60Hz), speed units (min⁻¹→rpm), power adds HP equivalent, ATEX classification removed, TEFC enclosure type added. “pressurising” → “pressurizing” (UK→US spelling). ATEX Ex eb IIC T3 Gb classification entirely removed. “perfluorelastomer” → “Perfluor”. Word-order change on cooling coil.

### 3.3 Drive Unit

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| 1× Mill motor 37 kW, 3 x 400 V, 50 Hz, 1500 min⁻¹ — II 2 G Ex db eb IIB T4 Gb, IP 55, with 3 PTC probes | 1× Mill motor 50 HP (37 kW), 3 x 460 V, 60 Hz, 1750 rpm, TEFC |
| 1× Pulley and antistatic V-belt for DYNO-DISC BC tip speed of 4.0 – 16.0 m/s | 1× Pulley and antistatic V-belt for DYNO-DISC BC tip speed of 4.0 – 16.0 m/s |

Difference: Major — Motor specs: power adds HP primary (50 HP), voltage 400V→460V, frequency 50Hz→60Hz, speed 1500 min⁻¹→1750 rpm. Full ATEX classification (Ex db eb IIB T4 Gb, IP 55) and PTC probes removed, replaced with TEFC enclosure type.

### 3.4 Sensors and Control Unit

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| 1× Explosion protection of sensors according Directive 2014/34/EU concerning equipment and protective systems intended for use in potentially explosive atmospheres — II 2 G Ex ia/ib IIC T4-T6 Gb | (not present — entire row removed) |
| Safety and control instruments consisting of: Two inductive proximity switches to prevent start of the mill when grinding container or WAB separation system is removed; Contact pressure gauge (MIN/MAX) 0 – 4 bar at the product inlet with inductive contacts and diaphragm pressure transducer with Tri-Clamp connection; Bimetal thermometer (MAX) 0 - 100 °C with pen sensor mounted in a robust protective tube; Two capacitive proximity switches for level control of the rinsing liquid; Contact pressure gauge (MIN/MAX) 0 - 6 bar with inductive contacts to control the pressure in the seal rinsing circuit; Instruments connected in a junction box for connection to electric circuit II 2 G Ex ia/ib IIC T4-T6 Gb, without relays | 1× Safety and control instruments consisting of: Two inductive proximity switches to prevent start of the mill when grinding container or WAB separation system is removed; Contact pressure gauge (MIN/MAX) 0 – 4 bar at the product inlet with inductive contacts and diaphragm pressure transducer with Tri-Clamp connection; Bimetal thermometer (MAX) 0 - 100 °C with pen sensor mounted in a robust protective tube; Two capacitive proximity switches for level control of the rinsing liquid; Contact pressure gauge (MIN/MAX) 0 - 6 bar with inductive contacts to control the pressure in the seal rinsing circuit; Flow monitor in buffer pressure line; Instruments connected in a junction box for connection to electric circuit, without relays |
| 1× Mill Control Unit WAB BASIC, mounted in the machine housing (inside wired on terminals) for control voltage 24 VDC, with: 3 ON/OFF-push buttons and pilot lamps; 2 potentiometers for the adjustment of the speed of the product pump and mill motors; EMERGENCY STOP switch; Ammeter (for connection to current transformer 0 – 1 A); Explosion protection II 2 G Ex db eb IIC T6 Gb, IP 65 | 1× Mill Control Unit WAB BASIC, mounted in the machine housing (inside wired on terminals) for control voltage 24 V DC, with: 3 ON/OFF-push buttons and pilot lamps; 2 potentiometers for speed adjustment of mill and product pump motors; EMERGENCY STOP switch |
| 1× Digital display for DYNO-DISC BC peripheral speed, mounted in the machine housing, with: Trigger 4-20 mA; Explosion protection II 2 G Ex ia IIC T6 Gb, IP 65 | 1× Digital display, mounted in the machine housing, with: Mill current (Amps); DYNO-DISC BC agitator peripheral speed (m/s); Product pump speed (rpm) |

Difference: Major — Four rows in CHE reduced to three in USA: - Row 1 (ATEX sensor directive declaration): entirely removed. - Row 2 (instruments): ATEX junction-box classification removed; “Flow monitor in buffer pressure line” added. - Row 3 (WAB BASIC): Ammeter and ATEX classification removed; wording slightly condensed. - Row 4 (digital display): CHE shows only peripheral speed with ATEX classification; USA shows three readouts (amps, speed, pump rpm) without ATEX. The ammeter functionality moved from the WAB BASIC unit to the digital display in the USA version.

### 3.5 Machine Housing & Support Material

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| 1× Machine housing in painted steel with stainless steel collecting dish for the grinding beads | 1× Painted steel machine housing with stainless steel collecting dish for the grinding beads |
| 1× Toolbox for dismantling of the mill parts | 1× Toolbox for dismantling of the mill parts |
| 1× Funnel made of stainless steel for filling of grinding beads | 1× Stainless steel funnel for filling of grinding beads |
| 1× Operational Manual in English | 1× Operational Manual in English |

Difference: Minor — Word-order rephrasing only (“Machine housing in painted steel” → “Painted steel machine housing”; “Funnel made of stainless steel” → “Stainless steel funnel”).

### 3.6 Price — Machine Execution

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| Price Machine Execution: CHF 89’750.00 | Price Machine Execution: USD 152,037.00 |

Difference: Major — Currency code CHF → USD. Thousand separator apostrophe → comma. Amount changes (89,750 → 152,037 — a factor of ~1.694).

## Section 4 — Product Pump

CHE title: “Product Pump FZ 1300”. USA title: “Product Pump (1200 l/h)”.

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| 1× Gear pump for throughputs up to 1300 l/h | 1× Pump for throughput up to 1200 l/h |
| Explosion protection of the product pump: Directive 2014/34/EU concerning equipment and protective systems intended for use in potentially explosive atmospheres — II 2 G Ex h IIB T3 Gb X | (ATEX declaration entirely removed) |
| The gear pump is mounted on a separate baseplate with legs, for the operation beside the DYNO-MILL UBM 20. | The pump is mounted on a separate baseplate with feet, for operation beside the DYNO-MILL UBM 20. |
| Pump specification | Pump specification |
| Motor 1.1 kW, 3 x 400 V, 50 Hz, 1500 min⁻¹ | Motor 1.5 HP (1.1 kW), 3 x 460 V AC, 60 Hz, 1750 rpm, TEFC(-XP) |
| II 2 G Ex db eb IIC T4 Gb, IP 55, with 3 PTC probes, 5 – 80 Hz | (ATEX classification removed) |
| Reducing gear | Rotational speed variable by frequency regulation and speed reducer gear |
| Single mechanical seal with O-rings (Kalrez) | Single mechanical seal with O-rings (Kalrez) |
| Temperature control by means of 1 PTC probe | (removed) |
| Nipple connection for product hose | Tri-Clamp connection for product hose |
| Filling hopper with ball valve mounted on the pump inlet | (removed) |
| (not present) | Max. pressure 4 bar |
| (not present) | Tubing between feed tank, pump, mill, and discharge is not included |

Difference: Major — Extensive changes: - Product name: “Gear pump FZ 1300” → “Pump (1200 l/h)” — model designation removed, lower throughput rating (1300→1200). - ATEX declarations (2 separate blocks) entirely removed. - Motor specs: US electrical standards (460V/60Hz/1750rpm/TEFC-XP), HP primary. - “Reducing gear” expanded to “Rotational speed variable by frequency regulation and speed reducer gear”. - PTC probe and filling hopper removed. - Connection type: nipple → Tri-Clamp. - Added max pressure spec and tubing-not-included disclaimer.

### 4.1 Price — Product Pump

| CHE (UBM 20 FU_FU) | USA (UBM 20 FCFC) |
| --- | --- |
| Price Product Pump: CHF 8’560.00 | Price Product Pump: USD 14,501.00 |

Difference: Major — Currency code CHF → USD. Thousand separator apostrophe → comma. Amount changes (8,560 → 14,501 — a factor of ~1.694).

## Summary of Differences

| Subsection | Assessment | Key Changes |
| --- | --- | --- |
| 1.1 Working Principle | Modest | Grammar fix, added screen-sizes row |
| 1.2 High Process Flexibility | Minor | “circulation” → “recirculation” |
| 1.3 Design | Modest | UL safety disclaimer added |
| 1.4 Cleaning & Maintenance | Minor | Plural → singular |
| 2.1 Conformity | Major | EU/ATEX declarations → unresolved US classification question |
| 2.2 Electrical Specifications | Major | 400V/50Hz → 460V/60Hz, added installed power row |
| 2.3 Grinding Unit | Modest | DIN metric specs → imperial equivalents, inlet/outlet merged |
| 2.4 Cooling Water Circulation | Minor | “BSPP” added, dash → “to” |
| 2.5 Machine Housing | Modest | Imperial primary units (inches, lbs) with metric in parens |
| 3.1 Grinding Unit | Minor | “milling chamber” → “milling”, remark row removed |
| 3.2 Shaft Sealing | Major | Motor specs changed, all ATEX removed, US spelling |
| 3.3 Drive Unit | Major | Motor specs changed, all ATEX removed, TEFC added |
| 3.4 Sensors and Control Unit | Major | ATEX row removed, ammeter moved to display, flow monitor added |
| 3.5 Machine Housing & Support | Minor | Word-order rephrasing only |
| 3.6 Price — Machine Execution | Major | CHF → USD, amount changed |
| 4 Product Pump (full) | Major | Model name changed, ATEX removed, motor specs, connection type, hopper removed, disclaimer added |
| 4.1 Price — Product Pump | Major | CHF → USD, amount changed |

### Patterns Observed

- ATEX removal is the single most pervasive change across sections 3 and 4. Every ATEX/Ex classification is stripped in the USA version. This is systematic and product-line-independent.

- Motor specs follow a consistent transformation: kW→HP(kW), 400V→460V, 50Hz→60Hz, min⁻¹→rpm, ATEX enclosure→TEFC.

- Units shift to imperial-primary with metric in parentheses for dimensions and weight (section 2) but remain metric-only for technical specs (pressure in bar, temperature in °C, flow in l/h).

- Price factor is consistent: both section 3 and section 4 prices show a ~1.694× multiplier (CHF → USD), suggesting the conversion uses FX × MU = 1.10 × 1.54 ≈ 1.694 or a similar combined factor.

- Conformity section is the only area with unresolved content (question mark in USA template) — this requires external input.