|  |  | Quotation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Quotation No. | : |  |
|  |  | Our Reference | : |  |
|  |  | Date | : |  |
|  |  | Customer No. | : |  |

| Your reference: |  |
| --- | --- |

Dear Name:

We thank you very much for your inquiry and are pleased to offer as follows:

| 1 | DYNO-MILL ECM-AP 10 Ceramic |
| --- | --- |
| Advanced Performance Series |  |
|  | 1 – Principal Characteristics 2 – General Technical Data 3 – Machine Execution 4 – Product Pump DM 2500 5 – Motor Starter Cabinet MSC-WAB VIEW 6 – Grinding Media 7 – Onsite Services 8 – Price Summary 9 – Options and Accessories 10 – Terms of Delivery |
| (Picture may not conform to the quotation) |  |

# 1 – Principal Characteristics

Working principle

- High-Efficiency Agitator Bead Mill using the “Patented” DYNO-ACCELERATOR grinding principle - The DYNO-ACCELERATOR ensures powerful hydraulic movement of the grinding beads and guarantees high & uniform energy input - DYNO-ACCELERATOR is designed to achieve the narrowest possible Particle Size Distribution

- Grinding media separation by means of the unique combination of the DYNO-ACCELERATOR and the new WAB separation system ADVANCED which permits the use of grinding beads from 0.1 mm - 1.25 mm

- Efficient heat transfer allows for superior cooling, permitting the processing of heat sensitive products

- Low power consumption due to highly efficient energy input (low carbon footprint)

Unique Process Flexibility

- Suitable for dispersion and wet grinding of low to high viscosity products - Wide range of energy input allows dispersing/grinding of a broad range of products at optimum parameters - Suitable for both, passage and recirculation processes - Unique batch size flexibility

Design

- Designed to ensure optimal ergonomics & user-friendly handling

- Reduced noise emission by mill motor placed inside the machine housing

- Easy operation & adjustment of operating parameters

- Built-in safety features

- Equipment is designed in accordance with applicable UL safety requirements but is not UL certified and will not bare the UL mark

Cleaning & Maintenance

- Self-cleaning effect of grinding chamber

- Short cleaning times using the smallest quantities of cleaning fluids, resulting in - faster product turnaround - reduced costs for cleaning fluids & recycling - less exposure to solvents by operator and environment

- Fast exchange of the screen (without the need to open the grinding container)

- Easy accessibility to all parts reduces handling time

- Double mechanical seal from WAB for long life and low maintenance costs

# 2 – General Technical Data

Conformity

| Conformity/ Protection: | General purpose/not rated Rated area/Class I/Div I |
| --- | --- |

Electrical specifications

| Operating voltage: | 460 V, 60 Hz (3 phases) |
| --- | --- |
| Installed power: | 50 HP (37 kW) |
| Control voltage: | 24 V DC |

Grinding unit

| Grinding cylinder volume: | 9.0 liter |
| --- | --- |
| Max. working pressure: | 4.0 bar |
| Working temperature: | 5 °C to 85 °C |
| Product inlet/outlet: | Tri-Clamp connection Ø 2” with Ø 1.25” hose connection and with PTFE seal |

| Cooling water circulation system |  |
| --- | --- |
| - Central cooling water connection for the cooling of the grinding unit and the rinsing liquid device: | With external thread connection G ½” |
| - Cooling water information |  |
| Flow rate: | 1500 – 2000 l/h |
| Pressure drop (with free outlet): | 2.5 – 4.0 bar |
| Max. cooling water pressure: | 6.0 bar |
| - Cooling water temperature range: | -15 °C to +50 °C |
| - Recommended cooling water temperature: | Below 10 °C |

Machine Housing

| Paint: | 2-Component paint of high quality, chemical resistant (in the colours grey, blue) |
| --- | --- |
| Dimensions L x W x H: | Appr. 60 x 40 x76 in (1520 x 1000 x 1920 mm) |
| Weight: | Appr. 2645 - 2866 lb (1200 - 1300 kg) |

# 3 – Machine Execution

3.1	Grinding Unit

| 1 | Stainless steel cooling cylinder with a removing device for fast & simple access to the milling chamber / DYNO-ACCELERATOR |  |
| --- | --- | --- |
| 1 | Grinding cylinder made of wear resistant silicon carbide |  |
| 1 | Stainless steel front cover with product outlet connection and drain plug |  |
| 1 | Set of DYNO-ACCELERATORs made of zirconium oxide / silicon carbide |  |
| 1 | WAB Separation system ADVANCED with central deaeration and screen disc of zirconium oxide |  |
| 1 | Cylinder Screen 0.25 mm, pure nickel, hard chromium plated |  |
| 1 | Set of spacers of zirconium oxide / silicon carbide including electrically conductive-/FDA compliant taper seals |  |
|  | Product contacted O-rings of Perfluor/FEP-O-Seal |  |
|  | Remaining parts in contact with the grinding beads in the wear area of zirconium oxide / silicon carbide |  |

3.2	Shaft Sealing

| 1 | Double mechanical seal from WAB Product side and atmosphere side sliding rings made of wear resistant silicon carbide with wear disc of zirconium oxide; fully sealed with Perfluor O-rings |  |
| --- | --- | --- |
| 1 | Rinsing and pressurizing of the shaft seal by means of separately driven, stainless steel gear pump type ZR 4030 Motor .33 HP (0.25 kW), 3 x 460 V, 60 Hz, 1000 rpm Stainless steel control valve to adjust operating pressure Rinsing liquid reservoir with cooling coil of stainless steel |  |
| 1 | Separate leakage container for the rinsing liquid |  |

3.3	Drive Unit

| 1 | Mill motor 50 HP (37 kW), 3 x 460 V, 60 Hz, 1500 |  |
| --- | --- | --- |
| 1 | Pulley and antistatic V-belt for DYNO-ACCELERATOR tip speed of 4.0 – 16.0 m/s |  |

3.4	Sensors and Control Unit

| 1 | Safety and control instruments consisting of - Two inductive proximity switches to prevent start of the mill when grinding container or WAB separation system is removed - Contact pressure gauge (MIN/MAX) 0 - 6 bar at the product inlet with inductive contacts and Tri-Clamp connection - Bimetal thermometer (MAX) 0 - 100 °C with pen sensor mounted in a robust protective tube - Two capacitive proximity switches for level control of the rinsing liquid - Contact pressure gauge (MIN/MAX) 0 - 6 bar with inductive contacts to control the pressure in the seal rinsing circuit - Instruments connected in a junction box for connection to electric circuit, without relays |  |
| --- | --- | --- |
| 1 | Mill Control Unit WAB VIEW mounted in the machine housing (inside wired on terminals) for control voltage 24 V DC - Touchdisplay (7" TFT) - USB interface for transmission of process parameters on USB-Stick - EMERGENCY STOP button mounted separately in the machine housing |  |

3.5	Machine Housing & Support material

| 1 | Painted steel machine housing |  |
| --- | --- | --- |
| 1 | Stainless steel collecting dish for the grinding beads |  |
| 1 | Toolbox for dismantling of the mill parts |  |
| 1 | Set of torque wrenches |  |
| 1 | Stainless steel funnel for filling of grinding beads |  |
| 1 | Operational Manual in English |  |

| Price Machine Execution zirconium oxide | USD | 404,324.00 |
| --- | --- | --- |

| Price Machine Execution silicon carbide | USD | 347,236.00 |
| --- | --- | --- |

# 4 – Product Pump

| 1 | Pneumatic driven membrane pump |  |  |
| --- | --- | --- | --- |
|  | Pneumatic driven membrane pump to be installed beside the DYNO-MILL ECM-AP 10. Pump specification - Range of capacity 100 – 2500 l/h - Stainless steel housing - Membranes of PTFE - Seals of PTFE - Needle valve for pressure regulation G ¼“ - Pulsation damper, stainless steel housing, membrane of PTFE - Solenoid valve and connecting box to be fitted on site Tubing between feed tank, pump, mill, and discharge is not included |  |  |
| Price Product Pump |  | USD | 18,746.00 |

# 5 – Motor Starter Cabinet MSC-WAB VIEW

| 1 | The motor starter cabinet has to be mounted outside of the explosion hazardous area. - PLC controller Siemens S7 with the necessary in- and output cards and software - Inside completely wired for connection to 3 x 460 V, 60 Hz - Prefuses - 24 V DC power supply for control voltage - Main switch - Contactor for the mill motor - Contactor for the product pump solenoid valve - Contactor for the rinsing liquid pump motor - Frequency converter for the mill motor - Relays for safety and control instruments - Ethernet port - Housing of painted sheet steel (grey) The motor starter cabinet is delivered without cable connection to the DYNO®-MILL and the product pump. |  |  |
| --- | --- | --- | --- |
| Price Motor Starter Cabinet |  | USD | 70,284.00 |

# 6 – Grinding Media

| 1 | High strength and wear resistant yttria-stabilized zirconia beads, for fine grinding applications with controlled particle size reduction - 0.6 mm Zirmil Y, quantity 60 kg (screen 0.25 mm) |  |  |
| --- | --- | --- | --- |
| Price Grinding Media |  | USD | x,xxx.00 |

# 7 – Onsite Services

| 1 | Two-day onsite startup and commissioning service, including: - Startup & commissioning & (1 day) – no charge - Operational & maintenance training (1 day) – no charge With charge: - Roundtrip travel time from Allendale, NJ to City, State - Travel expenses, per diem, transportation & accommodation |  |  |
| --- | --- | --- | --- |
| Price Onsite Services (*) |  | USD | x,xxx.00 |

(*) Please confirm dates with WAB at least 2 weeks prior to execution of site visit

# 8 – Price Summary

Execution zirconium oxide

| 3 – Machine Execution | USD | 404,324.00 |
| --- | --- | --- |
| 4 – Product Pump | USD | 18,746.00 |
| 5 – Motor Starter Cabinet | USD | 70,284.00 |
| Price for 1 DYNO-MILL ECM-AP 10 | USD | 493,354.00 |
| Packing, incl. wooden case | USD | 2,145.00 |
| Transportation | USD |  |
| 6 – Grinding Media | USD | x,xxx.00 |
| 7 – Onsite Services | USD | x,xxx.00 |
| 9 – Options and Accessories | USD | x,xxx.00 |
| TOTAL PRICE, FOB Allendale, packed | USD | x,xxx.00 |

(*) Sales tax may apply, please provide tax exempt certificate if applicable

Execution silicon carbide

| 3 – Machine Execution | USD | 347,236.00 |
| --- | --- | --- |
| 4 – Product Pump | USD | 18,746.00 |
| 5 – Motor Starter Cabinet | USD | 70,284.00 |
| Price for 1 DYNO-MILL ECM-AP 10 | USD | 436,266.00 |
| Packing, incl. wooden case | USD | 2,145.00 |
| Transportation | USD |  |
| 6 – Grinding Media | USD | x,xxx.00 |
| 7 – Onsite Services | USD | x,xxx.00 |
| 9 – Options and Accessories | USD | x,xxx.00 |
| TOTAL PRICE, FOB Allendale, packed | USD | x,xxx.00 |

(*) Sales tax may apply, please provide tax exempt certificate if applicable

# 9 – Options and Accessories

(not included in the total price)

9.1	Spare and Replacement Parts

| 1 | Cylinder screen 0.03 mm, pure nickel, hard chromium plated (Art. 246550) | USD | 2,522.00 |
| --- | --- | --- | --- |
| 1 | Cylinder screen 0.07 mm, pure nickel, hard chromium plated (Art. 235210) | USD | 2,522.00 |
| 1 | Cylinder screen 0.13 mm, pure nickel, hard chromium plated (Art. 235212) | USD | 2,522.00 |
| 1 | Cylinder screen 0.25 mm, pure nickel, hard chromium plated (Art. 235214) (price included in chapter 3 – Machine Execution) | USD | 2,522.00 |
| 1 | Cylinder screen 0.40 mm, pure nickel, hard chromium plated (Art. 235216) | USD | 2,522.00 |
| 1 | Cooling water multi sensor for max. temperature and min. cooling water flow | USD | 6,915.00 |

9.2	Documentation (*)

| 1 | IQ+OQ qualification documents | USD | 7,930.00 |
| --- | --- | --- | --- |
| 1 | FAT qualification documents, with realization by WAB, incl. measuring and calibration of instruments | USD | 12,560.00 |
| 1 | SAT qualification documents (without realization by WAB) | USD | 7,130.00 |

(*) These options may add time to the delivery

# 10 – Terms of Delivery

| Quotation: | FOB Allendale (INCOTERMS 2020), packed, including freight from Switzerland to the U.S., as well as the tariffs, duties, and taxes to import the goods. |
| --- | --- |
| Payment conditions: | • 40 % of the purchase order (PO) as first down payment, payable upon receipt of invoice • 50 % second down payment, invoiced and payable 30 days before shipping • 10 % NET 30 days date of shipping / invoiced and payable upon completion of onsite startup by WAB and operational acceptance by customer, not to exceed 60 days after delivery date Bank Information: Citibank, N.A., 1 Broadway, New York, NY 10004 Acct: 4978514689 - ABA: 021000089 Name on acct: WAB-US Corp |
| Lead time: | Approx. X weeks FOB Allendale from receipt of down payment and after clarification of all technical and commercial details |
| Documentation: | 1 Operation manual in English. |
| Warranty: | According to our ‘Standard Terms and Conditions of Sale’. The warranty excludes all wear parts. |
| Customer side services: | The customer is responsible for installing and connecting the machines according to our instructions. These services can be ordered separately from us. |
| Start-up: | Included in this proposal / Will be invoiced separately, if requested |
| Validity: | This offer is valid for 30 days. |
| Property reservation: | The goods remain our property without any restrictions, until fully paid. |
| Delivery terms: | According to our ‘Standard Terms and Conditions of Sale’. |
| Disclaimer: | Due to ongoing tariff changes, any duties or charges imposed after order confirmation are the customer’s responsibility and will be added to the final invoice. By ordering, the customer agrees to cover these costs. |
| Origin: | The above-mentioned goods are of Swiss origin (HS Code 8479.82). Changes reserved. |

WAB US Corp.

Name

Area Sales Manager

(phone)

(email)