|  |  |  | Quotation |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Quotation No. | : |  |
|  |  |  | Our Reference | : |  |
|  |  |  | Date | : |  |
|  |  |  | Customer No. | : |  |
| Your reference: |  |  |  |  |  |

Dear (Name):

We thank you very much for your inquiry and are pleased to offer as follows:

| 1 | DYNO-MILL UBM 20 |
| --- | --- |
|  | 1 – Principal Characteristics 2 – General Technical Data 3 – Machine Execution 4 – Product Pump (1200 l/h) 5 – Motor Starter Cabinet MSC-FC/FC 6 – Grinding Media 7 – Onsite Services 8 – Price Summary 9 – Options and Accessories 10 – Terms of Delivery |
| (Picture may not conform to the quotation) |  |

# 1 – Principal Characteristics

Working principle

- Agitator Bead Mill using the patented DYNO-DISC BC agitator discs

- The DYNO-DISC BC accelerates the grinding media along a well-defined path to build effective grinding circuits

- WAB grinding media separation by means of the combination of the DYNO-ROTOR BS (Bead Separation) and large WAB separation system UNIVERSAL. Permits the use of grinding beads from 0.2 mm – 2.5 mm

- Screens available: 0.07 mm for beads ≥ 0.2 mm, 0.13 mm for beads ≥ 0.4 mm, 0.25 mm for beads ≥ 0.8 mm, 0.40 mm for beads ≥ 1.2 mm, and 0.60 mm for beads ≥ 1.8 mm

- Excellent cooling properties permit the processing of heat-sensitive products

- High product throughput

- Lower power consumption due to highly efficient energy input (low carbon footprint)

High Process Flexibility

- Suitable for dispersion and wet grinding of low to high viscosity products - Wide range of energy input allows dispersing/grinding of a broad range of products at optimum parameters - Quick and easy definition of stable processing parameters - Suitable for both, passage and recirculation processes - High batch size flexibility

Design

- Designed to ensure optimal ergonomics & user-friendly handling

- Reduced noise emission by mill motor placed inside the machine housing

- Easy operation & adjustment of operating parameters

- Built-in safety features

- Equipment is designed in accordance with applicable UL safety requirements but is not UL certified and will not bare the UL mark

- Robust bead collecting channel integrated into the machine housing

Cleaning & Maintenance

- Short cleaning times

- Faster product turnaround

- Fast exchange of the screen without the need to open the grinding chamber

- Wear-resistant design reducing service costs

- Easy accessibility to machine parts

- Double mechanical seal from WAB for long life

# 2 – General Technical Data

Conformity

| Conformity / Protection: | General purpose / not rated or Rated environment / Class I / Div 1? |
| --- | --- |

Electrical specifications

| Operating voltage: | 460 V AC, 60 Hz (3 phases) |
| --- | --- |
| Installed power: | Approx. 50 HP (37 kW) |
| Control voltage: | 24 V DC |

Grinding unit

| Grinding cylinder volume: | 19.6 litres |
| --- | --- |
| Active grinding cylinder volume: | 17.4 litres |
| Max. working pressure: | 4 bar |
| Working temperature: | +5 °C to +85 °C |
| Product inlet / outlet: | Tri-Clamp connection Ø 2“(or equivalent) with 1.25” hose barb fitting and PTFE gasket |

| Cooling water circulation systems |  |
| --- | --- |

| Grinding container cooling |  |
| --- | --- |
| - Connection: | Hose connection Ø 16 mm |
| - Flow rate: | 800 – 2000 l/h |
| - Pressure drop (with free outlet): | 0.3 – 1.6 bar |
| - Cooling cylinder temperature range: | –15 °C to +50 °C |
| Buffer pressure unit |  |
| - Connection: | With external thread connection G ½” BSPP |
| - Flow rate: | 100 l/h |
| Cooling water |  |
| - Max. cooling water pressure: | 6 bar |
| - Recommended cooling water temperature: | Below 10 °C |

Machine Housing

| Paint: | 2-Component paint of high quality, chemical resistant (in the colours grey, blue) |
| --- | --- |
| Dimensions L x W x H: | Appr. 70 x 40 x 71 in (1785 x 1025 x 1800 mm) |
| Weight: | Appr. 2535 - 2755 lb (1150 - 1250 kg) |
| # 3 – Machine Execution |  |

3.1	Grinding Unit

| 1 | Cooling cylinder in stainless steel for fast access to the milling due to the ergonomic design |  |
| --- | --- | --- |
| 1 | Front cover, product inlet and product outlet of stainless steel |  |
| 1 | Drain plug of hardened stainless steel mounted in the front cover |  |
| 1 | Grinding cylinder made of hardened stainless steel |  |
| 1 | Set of DYNO-DISC BC and DYNO-ROTOR BS made of hardened stainless steel |  |
| 1 | Set of spacers of hardened stainless steel including special taper seals |  |
| 1 | WAB Separation system UNIVERSAL Cylindrical slot screen made of stainless steel, PVD-coated Gap width 0.25 mm Screen cover made of stainless steel, hard chrome plated |  |
|  | All remaining parts in contact with the grinding beads of hardened stainless steel |  |
|  | All product contacted O-rings of Viton/FEP-O-Seal |  |

3.2	Shaft Sealing

| 1 | Double mechanical seal from WAB Product side and atmosphere side sliding rings made of wear resistant Silicon Carbide; fully sealed with Perfluor O-rings |  |
| --- | --- | --- |
| 1 | Rinsing and pressurizing of the shaft seal by means of separately driven, stainless steel gear pump type ZR 4030 Motor 0.33 HP (0.25 kW), 3 x 460 V, 60 Hz, 1200 rpm, TEFC Operating pressure adjustable with stainless steel control valve Rinsing liquid reservoir with stainless steel cooling coil |  |
| 1 | Separate leakage container for the rinsing liquid |  |

3.3	Drive Unit

| 1 | Mill motor 50 HP (37 kW), 3 x 460 V, 60 Hz, 1750 rpm, TEFC |  |
| --- | --- | --- |
| 1 | Pulley and antistatic V-belt for DYNO-DISC BC tip speed of 4.0 – 16.0 m/s |  |

3.4	Sensors and Control Unit

| 1 | Safety and control instruments consisting of: - Two inductive proximity switches to prevent start of the mill when grinding container or WAB separation system is removed - Contact pressure gauge (MIN/MAX) 0 – 4 bar at the product inlet with inductive contacts and diaphragm pressure transducer with Tri-Clamp connection - Bimetal thermometer (MAX) 0 - 100 °C with pen sensor mounted in a robust protective tube - Two capacitive proximity switches for level control of the rinsing liquid - Contact pressure gauge (MIN/MAX) 0 - 6 bar with inductive contacts to control the pressure in the seal rinsing circuit - Flow monitor in buffer pressure line - Instruments connected in a junction box for connection to electric circuit, without relays |  |
| --- | --- | --- |
| 1 | Mill Control Unit WAB BASIC, mounted in the machine housing (inside wired on terminals) for control voltage 24 V DC, with - 3 ON/OFF-push buttons and pilot lamps - 2 potentiometers for speed adjustment of mill and product pump motors - EMERGENCY STOP switch |  |
| 1 | Digital display, mounted in the machine housing, with: - Mill current (Amps) - DYNO-DISC BC agitator peripheral speed (m/s) - Product pump speed (rpm) |  |

3.5	Machine Housing & Support material

| 1 | Painted steel machine housing with stainless steel collecting dish for the grinding beads |  |
| --- | --- | --- |
| 1 | Toolbox for dismantling of the mill parts |  |
| 1 | Stainless steel funnel for filling of grinding beads |  |
| 1 | Operational Manual in English |  |

| Price Machine Execution | USD | 152,037.00 |
| --- | --- | --- |

# 4 – Product Pump (1200 l/h)

| 1 | Pump for throughput up to 1200 l/h |  |  |
| --- | --- | --- | --- |
|  | The pump is mounted on a separate baseplate with feet, for operation beside the DYNO®-MILL UBM 20. Pump specification - Motor 1.5 HP (1.1 kW), 3 x 460 V AC, 60 Hz, 1750 rpm, TEFC(-XP) - Rotational speed variable by frequency regulation and speed reducer gear - Single mechanical seal with O-rings (Kalrez) - Tri-Clamp connection for product hose - Max. pressure 4 bar Tubing between feed tank, pump, mill, and discharge is not included |  |  |
| Price Product Pump |  | USD | 14,501.00 |

# 5 – Motor Starter Cabinet MSC-FC/FC

| 1 | Motor starter cabinet to be mounted beside the DYNO®-MILL / outside of the hazardous area, including: - Inside completely wired for connection to 460 V AC, 60 Hz, 3 phase - Prefuses - 24 V DC power supply for control voltage - Main switch - Contactor for the rinsing liquid pump motor - Frequency converter for the mill motor - Frequency converter for the product pump motor - Time counter - Relays for safety and control instruments - Siemens / Allen Bradley automation, or similar per vendor - NEMA 12 housing of painted in grey The motor starter cabinet is delivered without cable connection to the DYNO®-MILL and the product pump. |  |  |
| --- | --- | --- | --- |
| Price Motor Starter Cabinet |  | USD | 46,178.00 |

# 6 – Grinding Media

| 1 | High strength and wear resistant yttria-stabilized zirconia beads, for fine grinding applications with controlled particle size reduction - 0.6 mm Zirmil Y, quantity 60 kg (screen 0.25 mm) |  |  |
| --- | --- | --- | --- |
| Price Grinding Media |  | USD | x,xxx.00 |

# 7 – Onsite Services

| 1 | Two-day onsite startup and commissioning service, including: - Startup & commissioning & (1 day) – no charge - Operational & maintenance training (1 day) – no charge With charge: - Roundtrip travel time from Allendale, NJ to City, State - Travel expenses, per diem, transportation & accommodation |  |  |
| --- | --- | --- | --- |
| Price Onsite Services (*) |  | USD | x,xxx.00 |

(*) Please confirm dates with WAB at least 2 weeks prior to execution of site visit

# 8 – Price Summary

| 3 – Machine Execution | USD | 152,037.00 |
| --- | --- | --- |
| 4 – Product Pump | USD | 14,501.00 |
| 5 – Motor Starter Cabinet | USD | 46,178.00 |
| Price for 1 DYNO-MILL UBM 20 | USD | 212,716.00 |
| Transportation from Muttenz | USD |  |
| 6 – Grinding Media | USD | x,xxx.00 |
| 7 – Onsite Services | USD | x,xxx.00 |
| 9 – Options and Accessories | USD | x,xxx.00 |
| Packing, incl. wooden case | USD | 2,145.00 |
| TOTAL PRICE, FCA Muttenz, packed | USD | xxx,xxx.00 |

(*) Sales tax may apply, please provide tax exempt certificate if applicable

# 9 – Options and Accessories

(Not included in the total price)

9.1	Spare and Replacement Parts

| 1 | WAB cylindrical slot screen 0.07 mm, stainless steel, PVD-coated (art. 252159) | USD | 7,765.00 |
| --- | --- | --- | --- |
| 1 | WAB cylindrical slot screen 0.13 mm, stainless steel, PVD-coated (art. 250918) | USD | 7,765.00 |
| 1 | WAB cylindrical slot screen 0.25 mm, stainless steel, PVD-coated (art. 250916) (price included in chapter 3 – Machine Execution) | USD | 7,765.00 |
| 1 | WAB cylindrical slot screen 0.40 mm, stainless steel, PVD-coated (art. 250914) | USD | 7,765.00 |
| 1 | WAB cylindrical slot screen 0.60 mm, stainless steel, PVD-coated (art. 250907) | USD | 7,935.00 |
| 1 | Ball check valve DN 1½" with flange connection, mounted on the pump outlet | USD | 5,356.00 |
| 1 | Drain cock with screen 0.07, 0.13, 0.25 or 0.40 mm of stainless steel mounted in the front cover instead of drain plug Extra charge | USD | 1,176.00 |
| 1 | Set of torque wrenches (art. 241106) | USD | 2,919.00 |
| 1 | Grinding cylinder made of silicon carbide instead of hardened stainless steel Extra charge | USD | 36,530.00 |
| 1 | O-rings (static seals) for all product contacted parts made of Perfluor/FEP-O-Seal instead of standard Viton/FEP-O-Seal Extra charge | USD | 6,079.00 |

9.2	Documentation (*)

| 1 | IQ+OQ qualification documents | USD | 7,930.00 |
| --- | --- | --- | --- |
| 1 | FAT qualification documents, with realization by WAB, incl. measuring and calibration of instruments | USD | 12,560.00 |
| 1 | SAT qualification documents (without realization by WAB) | USD | 7,130.00 |

(*) These options may add time to the delivery

9.3	Alternate Product Pump

| Product Pump (2300 l/hr) |  |  |
| --- | --- | --- |

| 1 | Gear pump for throughputs up to 2300 l/h |  |  |
| --- | --- | --- | --- |
|  | The pump is mounted on a separate baseplate with feet, for operation beside the DYNO®-MILL UBM 20. Pump specification - Motor 2 HP (1.5 kW), 3 x 460 V, 60 Hz, 1750 rpm, TEFC(-XP) - Rotational speed variable by frequency regulation and speed reducer gear - Single mechanical seal with O-rings (Kalrez) - Tri-Clamp connection for product hose - Max. pressure 4 bar Tubing between feed tank, pump, mill, and discharge is not included |  |  |
| Price Product Pump |  | USD | 15,788.00 |

# 10 – Terms of Delivery

| Quotation: | FCA Muttenz (INCOTERMS 2020), packed, including freight from Switzerland to the U.S., as well as the tariffs, duties, and taxes to import the goods. |
| --- | --- |
| Payment conditions: | • 40 % of the purchase order (PO) as first down payment, payable upon receipt of invoice • 40 % second down payment, invoiced and payable 30 days before shipping • 20 % NET 30 days date of shipping / invoiced and payable upon completion of onsite startup by WAB and operational acceptance by customer, not to exceed 60 days after delivery date Bank Information: Citibank, N.A., 1 Broadway, New York, NY 10004 Acct: 4978514689 - ABA: 021000089 Name on acct: WAB-US Corp |
| Lead time: | Approx. X weeks FCA Muttenz from receipt of down payment and after clarification of all technical and commercial details |
| Documentation: | 1 Operation manual in English. |
| Warranty: | According to our ‘Standard Terms and Conditions of Sale’. The warranty excludes all wear parts. |
| Customer side services: | The customer is responsible for installing and connecting the machines according to our instructions. These services can be ordered separately from us. |
| Start-up: | Included in this proposal / Will be invoiced separately, if requested |
| Validity: | This offer is valid for 30 days. |
| Property reservation: | The goods remain our property without any restrictions, until fully paid. |
| Delivery terms: | According to our ‘Standard Terms and Conditions of Sale’. |
| Disclaimer: | Due to ongoing tariff changes, any duties or charges imposed after order confirmation are the customer’s responsibility and will be added to the final invoice. By ordering, the customer agrees to cover these costs. |
| Origin: | The above-mentioned goods are of Swiss origin (HS Code 8479.82). Changes reserved. |

WAB US Corp.

Name

Area Sales Manager

(phone)

(email)