|  |  | Quotation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Quotation No. | : |  |
|  |  | Our Reference | : |  |
|  |  | Date | : |  |

| Your reference: |  |
| --- | --- |

Dear Sirs

We thank you very much for your inquiry and are pleased to offer as follows:

| 1 | DYNO-MILL ECM-AP 10 ceramic |
| --- | --- |
| Advanced Performance Series |  |
|  | 1 – Principal Characteristics 2 – General Technical Data 3 – Machine Execution 4 – Product Pump DM 2500 5 – Motor Starter Cabinet MSC-WAB VIEW 6 – Price Summary 7 – Options and Accessories 8 – Terms of Delivery |
| (Picture does not conform exactly to the quotation) |  |

# 1 – Principal Characteristics

Working principle

- High-Efficiency Agitator Bead Mill using the “Patented” DYNO-ACCELERATOR grinding principle - The DYNO-ACCELERATOR ensure powerful hydraulic movement of the grinding beads and guarantee high & uniform energy input - DYNO-ACCELERATOR are designed to achieve the narrowest possible Particle Size Distribution

- Grinding media separation by means of the unique combination of the DYNO-ACCELERATOR and the new WAB separation system ADVANCED which permits the use of grinding beads from 0.1 mm - 1.25 mm

- Most efficient heat transfer allows for superior cooling, permitting the processing of heat-sensitive products

- Lowest power consumption due to highly efficient energy input (low carbon footprint)

Unique Process Flexibility

- Suitable for dispersion and wet grinding of low to high viscosity products - Wide range of energy input allows dispersing/grinding of a broad range of products at its optimum parameters - Suitable for both, passage- and circulation processes - Unique batch size flexibility

Design

- Designed to ensure optimal ergonomics & user-friendly handling

- Reduced noise emission by mill motor placed inside the machine housing

- Easy operation & adjustment of operation parameters

- Built-in safety features

Cleaning & Maintenance

- Self Cleaning effect of grinding chamber

- Short Cleaning Times using the smallest quantities of cleaning fluids, resulting in - faster product turnarounds - reduced costs for cleaning fluids & recycling - less exposure to solvents by operator and environment

- Fast exchange of the screen (without the need to open the grinding container)

- Easy accessibility to all parts reduces handling time to a minimum

- Double mechanical seal from WAB for long life and low maintenance costs

# 2 – General Technical Data

Conformity

| EC Declaration of Conformity: | covered by the Directive 2006/42/EC on machinery, Annex II A, and the ATEX directive 2014/34/EU, Annex VIII |
| --- | --- |
| Explosion protection of the machine: | II 2 G Ex h IIB T3 Gb |
| Protection: | IP 55 (acc. IEC 60529) |

Electrical specifications

| Operating voltage: | 400 V, 50 Hz (3 phases) |
| --- | --- |
| Control voltage: | 24 VDC |

Grinding unit

| Grinding cylinder volume: | 9.0 liter |
| --- | --- |
| Max. working pressure: | 4.0 bar |
| Working temperature: | 5 °C - 85 °C |
| Product inlet: | Tri-Clamp connection Ø 50.5 mm acc. DIN 32676 for tube acc. DIN 11866 line B DN/OD Ø 33.7 mm with hose connection Ø 31.8 mm and with PTFE seal |
| Product outlet: | Tri-Clamp connection Ø 50.5 mm acc. DIN 32676 for tube acc. DIN 11866 line B DN/OD Ø 33.7 mm with hose connection Ø 31.8 mm and with PTFE seal |

| Cooling water circulation system |  |
| --- | --- |
| - Central cooling water connection for the cooling of the grinding unit and the rinsing liquid device: | With external thread connection G ½” |
| - Cooling water information |  |
| Flow rate: | 1500 – 2000 l/h |
| Pressure drop (with free outlet): | 2.5 – 4.0 bar |
| Max. cooling water pressure: | 6.0 bar |
| - Cooling water temperature range: | -15 °C – +50 °C |
| - Recommended cooling water temperature: | Below 10 °C |

Machine Housing

| Paint: | 2-Component paint of high quality, chemical resistant (in the colours grey, blue) |
| --- | --- |
| Dimensions L x W x H (mm): | Approx. 1520 x 1000 x 1920 |
| Weight: | Approx. 1200 - 1300 kg |

# 3 – Machine Execution

3.1	Grinding Unit

| 1 | Cooling cylinder in stainless steel including a removing device for fast & simple access to the milling chamber / DYNO-ACCELERATOR due to the ergonomic design |  |
| --- | --- | --- |
| 1 | Grinding cylinder made of wear resistant silicon carbide |  |
| 1 | Front cover of stainless steel with product outlet connection and drain plug |  |
| 1 | Set of DYNO-ACCELERATOR made of zirconium oxide / silicon carbide |  |
| 1 | WAB Separation system ADVANCED with central deaeration and screen disc of zirconium oxide |  |
| 1 | Cylinder Screen 0.25 mm, pure nickel, hard chromium plated |  |
| 1 | Set of spacers of zirconium oxide / silicon carbide including electrically conductive-/FDA compliant taper seals |  |
|  | Product contacted O-rings of perfluorelastomer/FEP-O-Seal |  |
|  | Remaining parts in contact with the grinding beads in the wear area of zirconium oxide / silicon carbide |  |

3.2	Shaft Sealing

| 1 | Double mechanical seal from WAB Product side and atmosphere side sliding rings made of wear resistant silicon carbide with wear disc of zirconium oxide; fully sealed with perfluorelastomer O-rings |  |
| --- | --- | --- |
| 1 | Rinsing and pressurising of the shaft seal by means of separately driven, stainless steel gear pump type ZR 4030 Motor 0.25 kW, 3 x 400 V, 50 Hz, 1000 min-1 II 2 G Ex eb IIC T3 Gb, IP 55 Operating pressure adjustable with stainless steel control valve Rinsing liquid reservoir with cooling coil of stainless steel |  |
| 1 | Separate leakage container for the rinsing liquid |  |

3.3	Drive Unit

| 1 | Mill motor 37 kW, 3 x 400 V, 50 Hz, 1500 min-1 II 2 G Ex db eb IIB T4 Gb, IP 55, with 3 PTC probes |  |
| --- | --- | --- |
| 1 | Pulley and antistatic V-belt for DYNO-ACCELERATOR tip speed of 4.0 – 16.0 m/s |  |

3.4	Sensors and Control Unit

| 1 | Explosion protection of sensors according Directive 2014/34/EU concerning equipment and protective systems intended for use in potentially explosive atmospheres II 2 G Ex ia/ib IIC T4-T6 Gb |  |
| --- | --- | --- |
|  | Safety and control instruments consisting of - Two inductive proximity switches to prevent start of the mill when grinding container or WAB separation system is removed - Tube pressure transmitter in the mill inlet, with output signal 4-20 mA and tube shaped Tri-Clamp connection - Temperature transmitter (PT 100) in the mill outlet, with output signal 4-20 mA and pen sensor mounted in a robust protective tube - Two capacitive proximity switches for level control of the rinsing liquid - Tube pressure transmitter in the seal rinsing liquid circuit, with output signal 4-20 mA - Pressure gauge 0 - 6 bar with to control the pressure in the seal rinsing circuit, mounted in the machine housing - Instruments connected in a junction box for connection to electric circuit II 2 G Ex ia/ib IIC T4-T6 Gb, without relays |  |
| 1 | Mill Control Unit WAB VIEW Pro, mounted in the machine housing (inside wired on terminals) for control voltage 24 VDC - Touchdisplay (12" TFT) Explosion protection ATEX Zone 1 and Zone 21 Protection IP 66 Processor Intel® ATOM™ E3940 RAM 8 GB Hard disc 128 GB Software Windows 10 IoT Enterprise 2021 LTSC WinCC Runtime Advanced, Version: V17 - USB interface for transmission of process parameters USB-Stick - EMERGENCY STOP button mounted separately in the machine housing Explosion protection II 2 G Ex db eb IIC T6 Gb Protection IP 65 |  |

3.5	Machine Housing & Support material

| 1 | Machine housing in painted steel |  |
| --- | --- | --- |
| 1 | Stainless steel collecting dish for the grinding beads |  |
| 1 | Toolbox for dismantling of the mill parts |  |
| 1 | Set of torque wrenches |  |
| 1 | Funnel made of stainless steel for filling of grinding beads |  |
| 1 | Operational Manual in English |  |

| Price Machine Execution zirconium oxide | CHF | 242’730.00 |
| --- | --- | --- |

| Price Machine Execution silicon carbide | CHF | 209’030.00 |
| --- | --- | --- |

# 4 – Product Pump DM 2500

| 1 | Pneumatic driven membrane pump |  |  |
| --- | --- | --- | --- |
|  | Explosion protection of the product pump: Directive 2014/34/EU concerning equipment and protective systems intended for use in potentially explosive atmospheres II 2 GDc IIB T80°C Pneumatic driven membrane pump to be installed beside the DYNO-MILL ECM-AP 10. Pump specification - Range of capacity 100 – 2500 l/h - Stainless steel housing - Membranes of PTFE - Seals of PTFE - Needle valve for pressure regulation G ¼“ - Pulsation damper, stainless steel housing, membrane of PTFE - Solenoid valve and connecting box to be fitted on site |  |  |
| Price Product Pump |  | CHF | 11’066.00 |

# 5 – Motor Starter Cabinet MSC-WAB VIEW

| 1 | Important remark: The motor starter cabinet has to be mounted outside of the explosion hazardous area. The motor starter cabinet is delivered without cable connection to the DYNO®-MILL and the product pump. Protection: IP 55 (acc. IEC 60529) - PLC controller Siemens S7 with the necessary in- and output cards and software - Inside completely wired for connection to 3 x 400 V, 50 Hz - Prefuses - Transformer 400/230V for power supply of the mill control with 230 VAC / 24 VDC - Main switch - Contactor for the mill motor - Contactor for the product pump solenoid valve - Contactor for the rinsing liquid pump motor - Frequency converter for the mill motor - Relays for safety and control instruments - Ethernet port - Housing of painted sheet steel (grey) |  |  |
| --- | --- | --- | --- |
| Price Motor Starter Cabinet |  | CHF | 41’490.00 |

# 6 – Price Summary

Execution zirconium oxide

| 3 – Machine Execution | CHF | 242’730.00 |
| --- | --- | --- |
| 4 – Product Pump | CHF | 11'066.00 |
| 5 – Motor Starter Cabinet | CHF | 41'490.00 |
| Price for 1 DYNO-MILL ECM-AP 10 | CHF | 295’286.00 |
| Packing, incl. wooden case | CHF | 1’650.00 |
| TOTAL PRICE, FCA Muttenz, packed | CHF | 296’936.00 |

Execution silicon carbide

| 3 – Machine Execution | CHF | 209’030.00 |
| --- | --- | --- |
| 4 – Product Pump | CHF | 11'066.00 |
| 5 – Motor Starter Cabinet | CHF | 41'490.00 |
| Price for 1 DYNO-MILL ECM-AP 10 | CHF | 261’586.00 |
| Packing, incl. wooden case | CHF | 1’650.00 |
| TOTAL PRICE, FCA Muttenz, packed | CHF | 263’236.00 |

# 7 – Options and Accessories

(not included in the total price)

| 1 | Cylinder screen 0.03 mm, pure nickel, hard chromium plated (Art. 246550) | CHF | 890.00 |
| --- | --- | --- | --- |
| 1 | Cylinder screen 0.07 mm, pure nickel, hard chromium plated (Art. 235210) | CHF | 890.00 |
| 1 | Cylinder screen 0.13 mm, pure nickel, hard chromium plated (Art. 235212) | CHF | 890.00 |
| 1 | Cylinder screen 0.25 mm, pure nickel, hard chromium plated (Art. 235214) (price included in chapter 3 – Machine Execution) | CHF | 890.00 |
| 1 | Cylinder screen 0.40 mm, pure nickel, hard chromium plated (Art. 235216) | CHF | 890.00 |
| 1 | Cooling water multi sensor for max. temperature and min. cooling water flow | CHF | 2’440.00 |

| 1 | Audit Trail on HMI FDA 21 CFR Part 11 User Management Extra charge | CHF | 4'270.00 |
| --- | --- | --- | --- |

# 8 – Terms of Delivery

| Quotation: | FCA Muttenz (INCOTERMS 2020), packed, excluding duty and taxes to import the goods into the country of destination |
| --- | --- |
| Payment conditions: | 40 % at placement of purchase order 40 % before delivery 20 % at latest 30 days date of invoice / date of shipping documents Against irrevocable and confirmed Letter of Credit to our favour through our bank. Validity of L/C should be for the period of 6 weeks beyond the date of shipment. All bank charges, incl. confirmation commission in your country as well as in Switzerland to your account. Our bank address: UBS Switzerland AG, 4002 Basel, Switzerland Account No: 233-514570.01N BIC (Swift Code): UBSWCHZH80A IBAN: CH92 0023 3233 5145 7001 N |
| Delivery time: | Approx. weeks FCA Muttenz from receipt of down payment / L/C and after clarification of all technical and commercial details. |
| Customer side services: | The customer is responsible for installing and connecting the machine according to our instructions. The grinding media DYNO®-Beads to operate the agitator bead mill can be ordered with the machine or from our local representatives. |
| Start-up: | Will be invoiced separately from us or our representatives. |
| Validity: | This quotation is valid until end of XXXX. |
| Property reservation: | The goods remain our property without any restrictions, until fully paid. |
| Delivery terms: | According the document “General Conditions of Contract for the supply of machinery and spare part” in its current version provided by the following link on our web page https://www.wab-group.com/fileadmin/GC-WAB-AG.pdf |
| Origin: | The above mentioned goods are of Swiss origin (HS-Code 8479.82). |

WILLY A. BACHOFEN AG