|  |  | Quotation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Quotation No. | : |  |
|  |  | Our Reference | : |  |
|  |  | Date | : |  |

| Your reference: |  |
| --- | --- |

Dear Sirs

We thank you very much for your inquiry and are pleased to offer as follows:

| 1 | DYNO-MILL UBM 20 |
| --- | --- |
|  | 1 – Principal Characteristics 2 – General Technical Data 3 – Machine Execution 4 – Product Pump FZ 1300 5 – Motor Starter Cabinet MSC-FC/FC 6 – Price Summary 7 – Options and Accessories 8 – Terms of Delivery |
| (Picture does not conform exactly to the quotation) |  |

# 1 – Principal Characteristics

Working principle

- Agitator Bead Mill using the patented DYNO-DISC BC agitator discs

- The DYNO-DISC BC accelerate the grinding media along a well-defined path to build effective grinding circuits

- WAB grinding media separation by means of the combination of the DYNO-ROTOR BS (Bead Separation) and large WAB separation system UNIVERSAL. Permits the use of grinding beads from 0.2 mm – 2.5 mm

- Excellent cooling properties permit the processing of also heat-sensitive products

- High product throughputs

- Lower power consumption due to highly efficient energy input (low carbon footprint)

High Process Flexibility

- Suitable for dispersion and wet grinding of low to high viscosity products - Wide range of energy input allows dispersing/grinding of a broad range of products at its optimum parameters - Quick and easy definition of stable processing parameters - Suitable for both, passage- and circulation processes - High batch size flexibility

Design

- Designed to ensure optimal ergonomics & user-friendly handling

- Reduced noise emission by mill motor placed inside the machine housing

- Easy operation & adjustment of operation parameters

- Built-in safety features

- Robust beads collecting channel integrated in the machine housing

Cleaning & Maintenance

- Short cleaning times

- Faster product turnarounds

- Fast exchange of the screen without the need to open the grinding chamber

- Wear-resistant design reducing service costs

- Easy accessibility to machine parts

- Double mechanical seal from WAB for long life

# 2 – General Technical Data

Conformity

| EC Declaration of Conformity: | covered by the Directive 2006/42/EC on machinery, Annex II A, and the ATEX directive 2014/34/EU, Annex VIII |
| --- | --- |
| Explosion protection of the machine: | II 2 G Ex h IIB T3 Gb |
| Protection: | IP 55 (acc. IEC 60529) |

Electrical specifications

| Operating voltage: | 400 V, 50 Hz (3 phases) |
| --- | --- |
| Control voltage: | 24 VDC |

Grinding unit

| Grinding cylinder volume: | 19.6 litres |
| --- | --- |
| Active grinding cylinder volume: | 17.4 litres |
| Max. working pressure: | 4 bar |
| Working temperature: | +5 °C – +85 °C |
| Product inlet: | Tri-Clamp connection Ø 50.5 mm acc. DIN 32676 for tube acc. DIN 11866 line B DN/OD Ø 33.7 mm with hose connection Ø 31.8 mm and with PTFE seal |
| Product outlet: | Tri-Clamp connection Ø 50.5 mm acc. DIN 32676 for tube acc. DIN 11866 line B DN/OD Ø 33.7 mm with hose connection Ø 31.8 mm and with PTFE seal |

| Cooling water circulation systems |  |
| --- | --- |

| Grinding container cooling |  |
| --- | --- |
| - Connection: | Hose connection Ø 16 mm |
| - Flow rate: | 800 – 2000 l/h |
| - Pressure drop (with free outlet): | 0.3 – 1.6 bar |
| - Cooling cylinder temperature range: | –15 °C – +50 °C |
| Buffer pressure unit |  |
| - Connection: | With external thread connection G ½” |
| - Flow rate: | 100 l/h |
| Cooling water |  |
| - Max. cooling water pressure: | 6 bar |
| - Recommended cooling water temperature: | Below 10 °C |

Machine Housing

| Paint: | 2-Component paint of high quality, chemical resistant (in the colours grey, blue) |
| --- | --- |
| Dimensions L x W x H (mm): | Approx. 1785 x 1025 x 1800 |
| Weight: | Approx. 1150 - 1250 kg |

# 3 – Machine Execution

3.1	Grinding Unit

| 1 | Cooling cylinder in stainless steel for fast access to the milling chamber due to the ergonomic design |  |
| --- | --- | --- |
| 1 | Front cover, product inlet and product outlet of stainless steel |  |
| 1 | Drain plug of hardened stainless steel mounted in the front cover |  |
| 1 | Grinding cylinder made of hardened stainless steel |  |
| 1 | Set of DYNO-DISC BC and DYNO-ROTOR BS made of hardened stainless steel |  |
| 1 | Set of spacers of hardened stainless steel including special taper seals |  |
| 1 | WAB Separation system UNIVERSAL Cylindrical slot screen made of stainless steel, PVD-coated Gap width 0.25 mm Screen cover made of stainless steel, hard chrome plated |  |
|  | Remark to grinding unit: |  |
|  | All remaining parts in contact with the grinding beads of hardened stainless steel |  |
|  | All product contacted O-rings of Viton/FEP-O-Seal |  |

3.2	Shaft Sealing

| 1 | Double mechanical seal from WAB Product side and atmosphere side sliding rings made of wear resistant Silicon Carbide; fully sealed with perfluorelastomer O-rings |  |
| --- | --- | --- |
| 1 | Rinsing and pressurising of the shaft seal by means of separately driven, stainless steel gear pump type ZR 4030 Motor 0.25 kW, 3 x 400 V, 50 Hz, 1000 min-1 II 2 G Ex eb IIC T3 Gb, IP 55 Operating pressure adjustable with stainless steel control valve Rinsing liquid reservoir with cooling coil of stainless steel |  |
| 1 | Separate leakage container for the rinsing liquid |  |

3.3	Drive Unit

| 1 | Mill motor 37 kW, 3 x 400 V, 50 Hz, 1500 min-1 II 2 G Ex db eb IIB T4 Gb, IP 55, with 3 PTC probes |  |
| --- | --- | --- |
| 1 | Pulley and antistatic V-belt for DYNO-DISC BC tip speed of 4.0 – 16.0 m/s |  |

3.4	Sensors and Control Unit

| 1 | Explosion protection of sensors according Directive 2014/34/EU concerning equipment and protective systems intended for use in potentially explosive atmospheres II 2 G Ex ia/ib IIC T4-T6 Gb |  |
| --- | --- | --- |
|  | Safety and control instruments consisting of - Two inductive proximity switches to prevent start of the mill when grinding container or WAB separation system is removed - Contact pressure gauge (MIN/MAX) 0 – 4 bar at the product inlet with inductive contacts and diaphragm pressure transducer with Tri-Clamp connection - Bimetal thermometer (MAX) 0 - 100 °C with pen sensor mounted in a robust protective tube - Two capacitive proximity switches for level control of the rinsing liquid - Contact pressure gauge (MIN/MAX) 0 - 6 bar with inductive contacts to control the pressure in the seal rinsing circuit - Instruments connected in a junction box for connection to electric circuit II 2 G Ex ia/ib IIC T4-T6 Gb, without relays |  |
| 1 | Mill Control Unit WAB BASIC, mounted in the machine housing (inside wired on terminals) for control voltage 24 VDC, with - 3 ON/OFF-push buttons and pilot lamps - 2 potentiometers for the adjustment of the speed of the product pump and mill motors - EMERGENCY STOP switch - Ammeter (for connection to current transformer 0 – 1 A) - Explosion protection II 2 G Ex db eb IIC T6 Gb, IP 65 |  |
| 1 | Digital display for DYNO-DISC BC peripheral speed, mounted in the machine housing, with - Trigger 4-20 mA - Explosion protection II 2 G Ex ia IIC T6 Gb, IP 65 |  |

3.5	Machine Housing & Support material

| 1 | Machine housing in painted steel with stainless steel collecting dish for the grinding beads |  |
| --- | --- | --- |
| 1 | Toolbox for dismantling of the mill parts |  |
| 1 | Funnel made of stainless steel for filling of grinding beads |  |
| 1 | Operational Manual in English |  |

| Price Machine Execution | CHF | 89’750.00 |
| --- | --- | --- |

# 4 – Product Pump FZ 1300

| 1 | Gear pump for throughputs up to 1300 l/h |  |  |
| --- | --- | --- | --- |
|  | Explosion protection of the product pump: Directive 2014/34/EU concerning equipment and protective systems intended for use in potentially explosive atmospheres II 2 G Ex h IIB T3 Gb X The gear pump is mounted on a separate baseplate with legs, for the operation beside the DYNO®-MILL UBM 20. Pump specification - Motor 1.1 kW, 3 x 400 V, 50 Hz, 1500 min-1 II 2 G Ex db eb IIC T4 Gb, IP 55, with 3 PTC probes, 5 – 80 Hz - Reducing gear - Single mechanical seal with O-rings (Kalrez) - Temperature control by means of 1 PTC probe - Nipple connection for product hose - Filling hopper with ball valve mounted on the pump inlet |  |  |
| Price Product Pump |  | CHF | 8’560.00 |

# 5 – Motor Starter Cabinet MSC-FC/FC

| 1 | Important remark: The motor starter cabinet has to be mounted outside of the explosion hazardous area. The motor starter cabinet is delivered without cable connection to the DYNO®-MILL and the product pump. Protection: IP 55 (acc. IEC 60529) - Inside completely wired for connection to 3 x 400 V, 50 Hz - Prefuses - Transformer 400 V/24 VDC for control voltage - Main switch - Current transformer for ammeter - Contactor for the mill motor - Contactor for the pump motor - Contactor for the rinsing liquid pump motor - Frequency converter for the mill motor - Frequency converter for the product pump motor - Time counter - Relays for safety and control instruments - Housing of painted sheet steel (grey) |  |  |
| --- | --- | --- | --- |
| Price Motor Starter Cabinet |  | CHF | 27’260.00 |

# 6 – Price Summary

| 3 – Machine Execution | CHF | 89'750.00 |
| --- | --- | --- |
| 4 – Product Pump | CHF | 8'560.00 |
| 5 – Motor Starter Cabinet | CHF | 27'260.00 |
| Price for 1 DYNO-MILL UBM 20 | CHF | 125’570.00 |
| Packing, incl. wooden case | CHF | 1’650.00 |
| TOTAL PRICE, FCA Muttenz, packed | CHF | 127’220.00 |

# 7 – Options and Accessories

(not included in the total price)

| 1 | WAB cylindrical slot screen 0.07 mm, stainless steel, PVD-coated (art. 252159) | CHF | 2'740.00 |
| --- | --- | --- | --- |
| 1 | WAB cylindrical slot screen 0.13 mm, stainless steel, PVD-coated (art. 250918) | CHF | 2'740.00 |
| 1 | WAB cylindrical slot screen 0.25 mm, stainless steel, PVD-coated (art. 250916) (price included in chapter 3 – Machine Execution) | CHF | 2'740.00 |
| 1 | WAB cylindrical slot screen 0.40 mm, stainless steel, PVD-coated (art. 250914) | CHF | 2'740.00 |
| 1 | WAB cylindrical slot screen 0.60 mm, stainless steel, PVD-coated (art. 250907) | CHF | 2’800.00 |
| 1 | Ball check valve DN 1½" with flange connection, mounted on the pump outlet | CHF | 1’890.00 |
| 1 | Drain cock with screen 0.07, 0.13, 0.25 or 0.40 mm of stainless steel mounted in the front cover instead of drain plug Extra charge | CHF | 415.00 |
| 1 | Set of torque wrenches (art. 241106) | CHF | 1’030.00 |
| 1 | Grinding cylinder made of silicon carbide instead of hardened stainless steel Extra charge | CHF | 12’890.00 |
| 1 | O-rings (static seals) for all product contacted parts made of perfluorelastomer/FEP-O-Seal instead of standard Viton/FEP-O-Seal Extra charge | CHF | 2’145.00 |

| Product Pump FZ 2500 |  |  |
| --- | --- | --- |

| 1 | Gear pump for throughputs up to 2500 l/h |  |  |
| --- | --- | --- | --- |
|  | Explosion protection of the product pump: Directive 2014/34/EU concerning equipment and protective systems intended for use in potentially explosive atmospheres II 2 G Ex h IIB T3 Gb X The gear pump is mounted on a separate baseplate with legs, for the operation beside the DYNO®-MILL UBM 20. Pump specification - Motor 1.5 kW, 3 x 400 V, 50 Hz, 1500 min-1 II 2 G Ex db eb IIC T4 Gb, IP 55, with 3 PTC probes, 5 – 80 Hz - Reducing gear - Single mechanical seal with O-rings (Kalrez) - Temperature control by means of 1 PTC probe - Product connections with hose stem - Filling hopper with ball valve mounted on the pump inlet |  |  |
| Price Product Pump |  | CHF | 9’320.00 |

# 8 – Terms of Delivery

| Quotation: | FCA Muttenz (INCOTERMS 2020), packed, excluding duty and taxes to import the goods into the country of destination |
| --- | --- |
| Payment conditions: | 40 % at placement of purchase order 40 % before delivery 20 % at latest 30 days date of invoice / date of shipping documents Against irrevocable and confirmed Letter of Credit to our favour through our bank. Validity of L/C should be for the period of 6 weeks beyond the date of shipment. All bank charges, incl. confirmation commission in your country as well as in Switzerland to your account. Our bank address: UBS Switzerland AG, 4002 Basel, Switzerland Account No: 233-514570.01N BIC (Swift Code): UBSWCHZH80A IBAN: CH92 0023 3233 5145 7001 N |
| Delivery time: | Approx. weeks FCA Muttenz from receipt of down payment / L/C and after clarification of all technical and commercial details. |
| Customer side services: | The customer is responsible for installing and connecting the machine according to our instructions. The grinding media DYNO®-Beads to operate the agitator bead mill can be ordered with the machine or from our local representatives. |
| Start-up: | Will be invoiced separately from us or our representatives. |
| Validity: | This quotation is valid until end of XXXX. |
| Property reservation: | The goods remain our property without any restrictions, until fully paid. |
| Delivery terms: | According the document “General Conditions of Contract for the supply of machinery and spare part” in its current version provided by the following link on our web page https://www.wab-group.com/fileadmin/GC-WAB-AG.pdf |
| Origin: | The above mentioned goods are of Swiss origin (HS Code 8479.82). |

WILLY A. BACHOFEN AG